Annex No. 2

TECHNICAL SPECIFICATION

PURCHASE OF INDUSTRIAL UNIVERSAL GRATER (NUTS, CHEESE, CHOCOLATE)

place, date

The following table:

| Parameter | Requirement |
| --- | --- |
| THE PARTICIPANT shall provide information or proof against each requirement. |  |
| Quantity | 1 pc. – Industrial universal grating machine with mobile base and full set of accessories. |
| Brand, model offered | Brand name (for identification purposes) and model name (for identification purposes). |
| Purpose | Heavy-duty industrial processing (grating, granulating, shredding) of nuts, hard and soft cheeses, chocolate, dry bread, and vegetables in a commercial food production environment. The machine must utilize a pre-rotation or equivalent gentle processing principle to prevent oily products (e.g., nuts) from being crushed or releasing oil during grating. |
| Performance / Capacity | • Processing capacity: 300 – 500 kg/h (depending on product type and consistency). • The system must be capable of continuous operation for batch processing. |
| Construction and Materials | • Material: Entire housing, grinding chamber, and base table must be made of Stainless Steel (minimum AISI 304 or equivalent). • Design: Vertical construction with a top-loading funnel. The grinding head and functional parts must be removable for cleaning without tools (dishwasher safe components). • Protection Class: Minimum IP 26 or higher to ensure protection against water splashes during cleaning. |
| Overall Dimensions (L × W × H) | External dimensions (including base table) must be within the following logical ranges to fit the designated workspace: • Length: 650 – 750 mm. • Width: 650 – 750 mm. • Height: 1350 – 1450 mm. |
| Weight | Machine weight (including stand) to ensure stability during heavy operation: • Range: 75 – 90 kg. |
| Electrical System | • Voltage: 3 x 380-420 V / 50 Hz (3P + N + E) . • Motor Power: 1.2 – 1.8 kW. • Must include overload protection and emergency stop integration. |
| Filling Opening | Large feed hopper to accommodate bulk product input. • Opening dimensions: approx. 150 mm diameter or rectangular equivalent (approx. 150 x 66 mm range). |
| Grating Technology | • System must use a rotational wing/impeller system that presses product against a stationary or rotating drum. • Must support "cut not crush" technology to ensure high-quality granulation of chocolate and nuts without melting or clumping. |
| Included Accessories (Mandatory) | 1. Mobile Base: Stainless steel mobile table with locking castors, designed to house a collection container (GN 1/1 compatible) underneath the discharge. 2. Collection Container: GN 1/1 (or similar) polycarbonate or stainless steel container included. 3. Full Set of Grating Drums (Complete Range): The offer must include a complete set of interchangeable grating inserts to ensure full universality. Minimum 20 different inserts must be included, covering: • Fine grating: Types 000, 00, 0, 1, 2, 3 (for nuts, breadcrumbs, hard cheese). • Medium grating: Types 4, 6, 7, 8, 9, 10 (for pizza cheese, vegetables). • Coarse/Flake grating: Types 11, 12, 13, 14, 15, 16, 17, 18 (for soft cheese, chocolate flakes). • Specialty cuts: Types 19 through 25 (Rösti, strips, shavings). 4. Impellers/Wings: Must include both "Neutral" (standard) and "Aggressive" (for harder products) rotating wings. |
| Safety Features | • Emergency stop switch prominent on the housing . • Safety micro-switches on the hopper and grinding head (machine must stop if opened). • Motor overload protection. |
| Hygiene & Cleaning | • "Hygienic Design" certification or equivalent. • All contact parts (drum, wing, hopper, discharge) must be removable and dishwasher safe. |
| Documentation | To be supplied in Latvian or English: user manual, technical passport, electric diagram, maintenance instructions, EC Declaration of Conformity. |
| Compliance and CE marking | Equipment must comply with EU Machinery Directive 2006/42/EC, Low Voltage Directive, and food contact materials legislation (EC 1935/2004). CE marking mandatory. |
| Guarantee | Minimum 24 months full manufacturer or supplier warranty. |

Name, surname, position

signature