## Annex No. 2

## Technical Specification

### Purchase of Industrial Vacuum Mixing and Tumbling Machines

| Parameter | Requirement |
| --- | --- |
| THE PARTICIPANT shall provide information or proof against each requirement. |  |
| Quantity | 2 pcs. – Industrial Vacuum Mixing and Tumbling Machines. |
| Purpose | Designed for massaging, marinating, tumbling, blending, and mixing meat, poultry, and fish products. Must ensure high product quality, even marinade distribution, and maximum marinade absorption under vacuum conditions. |
| Drum (Mixing Chamber) Specifications |  |
| * Nominal Volume | 240 L – 260 L. |
| * Working Capacity | 20 kg – 160 kg (depending on product density). |
| * Drum Type | Closed drum with the ability to create and maintain a vacuum during the mixing process. |
| * Drum Material | Stainless Steel (AISI 304 or equivalent). |
| External Dimensions (Per Unit) |  |
| * External Width | 110 cm – 120 cm. |
| * External Depth | 90 cm – 100 cm. |
| * External Height | 135 cm – 140 cm. |
| * Weight | 140 kg – 160 kg. |
| Technology and Control |  |
| * Speed Regulation | Gear motor with variable adjustable speed for mixing paddle/arm rotation. |
| * Control | Touch Screen Control (PLC control) with the capability to save a minimum of 99 recipes. |
| * Mixing Elements | The drum must have integrated mixing elements (paddles) for smooth and intensive massaging. |
| Mandatory Optional Equipment (Included) |  |
| * Bowl Tilting | Automatic bowl tilting and positioning system. |
| * Mixing Arm Accessory | Mixing arm including removable scraper with a press-button attachment mechanism. |
| * Loading System | Mandatory: Hydraulic loading system or mast type loading system, adapted for a 200-liter (DIN 9797) standard container (Eurobin). |
| Vacuum System |  |
| * Vacuum Type | Mandatory integrated vacuum pump system. |
| Complete Functional Set (Mandatory) | The Supplier must fully complete the equipment, understanding its intended purpose (meat processing and marinating). If further information is required regarding detailed completion, the Supplier must contact the procurement officer/person responsible for the purchase. The offer must explicitly include: |
|  | • The complete base equipment (with all sealing systems, motors, and controls). |
|  | • All three specified mandatory optional equipment items. |
|  | • Installation materials and final utility connections. |
|  | • Commissioning and comprehensive staff training. |
| Documentation | To be supplied in Latvian or English: user manual, electric diagram, maintenance instructions, and CE Declaration of Conformity. |
| Guarantee | Minimum 24 months full manufacturer or supplier warranty. |