# Annex No. 2

TECHNICAL SPECIFICATION

PURCHASE OF INFEED CONVEYOR

place, date

| Parameter | Requirement |
| --- | --- |
| THE PARTICIPANT shall provide information or proof against each requirement. |  |
| Quantity | 1 pc. – Stainless steel hygienic infeed conveyor for delivering trays/containers into spiral freezer. |
| Brand, model offered | Brand name (for identification purposes) and model name (for identification purposes). |
| Purpose | Conveyor shall be designed to transport C-PET trays or containers with ready meals weighing 200–450 g each, feeding products into a spiral freezer at a controlled and stable rate. |
| Overall dimensions (L × W × H) | Conveyor dimensions must be within the following limits: length 2900–3100 mm; width 850–950 mm; height 900–1000 mm. |
| Construction and materials | Conveyor frame, supports, guards and contact surfaces must be manufactured from stainless steel minimum AISI 304 or higher. All hygienic design principles must be maintained: smooth welds, slope for drainage, no hidden crevices. |
| Belt type | Removable hygienic belt suitable for food industry use. Belt must allow quick removal without tools (tool-less removal) for cleaning and inspection. Belt material must comply with EU Regulation (EC) No. 1935/2004. |
| Belt width | Belt width must be within 350–450 mm, ensuring compatibility with C-PET trays used for spiral freezer feeding. |
| Load capacity | Conveyor must support at least 2.5 kg per tray and continuous load comparable to spiral freezer feeding requirements. |
| Speed control | Conveyor must include variable speed control (VFD or equivalent), allowing synchronisation with spiral freezer infeed rate. Speed adjustment range must be suitable for 450–550 kg/h production flow. |
| Drive system | Motor and gearbox suitable for continuous industrial operation. Drive must be positioned to avoid contamination risk. Drive components must have at least IP65 protection or higher. |
| Hygienic design | Conveyor must follow hygienic design standards: open frame, easy access for cleaning, sloped surfaces, no hollow bodies unless sealed. Drainage concept must allow quick water removal. |
| Safety features | Mandatory emergency stop button, belt guarding, safe access design. Must comply with EU industrial safety requirements. |
| Installation | Conveyor must be delivered fully assembled except for utility connection. Height and angle must allow seamless transfer into the spiral freezer infeed zone. |
| Complete functional set (mandatory) | All components required for full operation (belt, motor, gearbox, supports, speed control panel, guards, legs, feet, cable set if applicable) must be included in the offered price. |
| List of supplied components | THE PARTICIPANT must include a detailed list of all supplied parts and accessories. |
| Documentation | To be supplied in Latvian or English: declaration of conformity, equipment passport, user manual, maintenance instructions, electrical diagram. |
| Compliance and CE marking | Conveyor must comply with relevant EU machinery, LVD and EMC directives. CE marking mandatory. |
| Guarantee | Minimum 24 months full manufacturer or supplier warranty. |

Name, surname, position

signature