Annex No. 2

TECHNICAL SPECIFICATION

PURCHASE OF MULTI-PURPOSE INDUSTRIAL PRESSURE COOKER (BRAISING PAN)

place, date

The following table:

| Parameter | Requirement |
| --- | --- |
| THE PARTICIPANT shall provide information or proof against each requirement. |  |
| Quantity | 2 pcs. – Industrial multi-purpose pressure cooker / braising pan. |
| Brand, model offered | Brand name (for identification purposes) and model name (for identification purposes). |
| Purpose | Versatile industrial cooking equipment capable of boiling, braising, frying, steaming, and pressure cooking in a single unit. Designed for heavy-duty catering and food production. |
| Capacity | • Volume: 170 – 190 litres • Cooking Surface Area: 80 – 90 dm². |
| Pressure Cooking System | • Lid Type: Pressure lid equipped with double opening/closing manoeuvring system for safety. • Pressure Range: Capable of cooking at 0.2 – 0.4 bar to reduce cooking times and save energy. • Condensation: Automatic condensation discharge system included. |
| Performance & Temperature | • Temperature Range: Adjustable from 50°C to 250°C. • Cooking Modes: Boil, fry, pressure cook, brown, slow/soft cooking. |
| Heating System (Electric) | • Type: Direct electric heating via armoured heating elements (INCOLOY-800 alloy or equivalent). • Control: Electronic board control with thermocouple system for precise temperature detection and high reaction speed. • Power: 25 kW – 30 kW . • Voltage: 400V / 3 ph / 50 Hz (3N+PE). |
| Construction | • Material: Entirely made of stainless steel (AISI 304 structure). • Pan Lifting: Motorised tilting system. • Lid: Double-wall insulated lid to reduce heat dispersion. |
| Control System | • Interface: Touchscreen control panel (e.g., Firex Touch Control or equivalent). • Functions: Manual mode, recipe programming, HACCP data backup, USB connection, multi-probe management. • Probes: Must include 3 temperature probes: 1. Bottom/Heating element probe, 2. Pan/Product contact probe, 3. Core/Pin probe for product core temperature. |
| Water Filling | • System: Automatic water filling into the pan controlled by a volumetric doser (litre counter) integrated into the control panel. |
| Dimensions | • Width: 1950 – 2050 mm • Depth: 1100 – 1150 mm • Height: 1000 – 1050 mm |
| Complete Functional Set (Mandatory) | The supplier is responsible for providing a fully functional unit with all necessary components for immediate operation, including but not limited to: • Core probe (pin probe) • Automatic water filling system • Motorised tilting • Pressure lid safety system • Cleaning shower/hose (if applicable/recommended) • Installation and commissioning accessories |
| Documentation | To be supplied in Latvian or English: user manual, electric diagram, maintenance instructions, CE Declaration of Conformity. |
| Compliance | Equipment must comply with EU Machinery Directive 2006/42/EC, Pressure Equipment Directive (PED), and food contact materials legislation. CE marking mandatory. |
| Guarantee | Minimum 24 months full manufacturer or supplier warranty. |

Name, surname, position

signature