Annex No. 2

TECHNICAL SPECIFICATION

PURCHASE OF REFRIGERATED FISH STORAGE CABINET

place, date

The following table:

| Parameter | Requirement |
| --- | --- |
| THE PARTICIPANT shall provide information or proof against each requirement. |  |
| Quantity | 5 pcs. – Vertical refrigerated cabinet specifically designed for fresh fish storage. |
| Brand, model offered | Brand name (for identification purposes) and model name (for identification purposes). |
| Purpose | Storage of fresh fish and seafood at specific negative/positive temperatures to prevent spoilage and dehydration. |
| Temperature Range | • Working Temperature: Adjustable range covering -4°C to +4°C to ensure optimal preservation of fresh fish. |
| Capacity & Storage | • Storage Levels: Designed to hold at least 7 levels of fish containers. • Container Size: Compatible with 600x400 mm containers/bins. • Included Containers: The unit must be supplied with a full set of 7 x specialty fish containers (stainless steel or food-grade plastic with perforated bottoms for drainage). |
| Construction | • Material: Interior and exterior made of high-quality Stainless Steel to resist corrosion from salts and fluids. • Insulation: High-density polyurethane, thickness 50 mm or higher • Drain: Integrated drainage system at the bottom for the evacuation of waste liquids/ice melt, crucial for fish hygiene. • Protection: IPX5 water protection rating or equivalent. |
| Cooling System | • Type: Top-mounted refrigeration unit. • Refrigerant: Environmentally friendly hydrocarbon R290 (ECO mode capable). • Climate Class: 4 (up to 30°C ambient). • Defrost: Automatic electric defrosting. |
| Control System | • Interface: Electronic control with a capacitive touch screen minimum 3 inches). • Functions: Temperature display, HACCP alarm history (optional but preferred), ECO mode. |
| Dimensions | • Width: 640 mm – 760 mm. • Depth: 700 mm – 820 mm. • Height: 1980 mm – 2020 mm. |
| Door | • Type: Solid stainless steel door. • Features: Self-closing mechanism, stay-open feature at 90°, ergonomic full-length handle. |
| Complete Functional Set (Mandatory) | The supplier is responsible for providing a fully functional unit ready for immediate use. The set must include: • 1 x Refrigerated Cabinet Unit • 7 x Fish Storage Containers (600x400 mm) • Adjustable stainless steel legs (130-200 mm) • Power cable and plug |
| Documentation | To be supplied in Latvian or English: user manual, electric diagram, maintenance instructions, CE Declaration of Conformity. |
| Compliance | Equipment must comply with EU Machinery Directive 2006/42/EC, Low Voltage Directive, and food contact materials legislation. CE marking mandatory. |
| Guarantee | Minimum 24 months full manufacturer or supplier warranty. |

Name, surname, position

signature