# Annex No. 2

TECHNICAL SPECIFICATION

PURCHASE OF ROLL-IN COMBI STEAMER, 20×GN1/1 (TROLLEY TYPE)

place, date

| Parameter | Requirement |
| --- | --- |
| THE PARTICIPANT shall provide information or proof against each requirement. |  |
| Quantity | 4 pcs. – Industrial roll-in combi steamer for high-capacity cooking, steaming, regeneration and mixed-mode processes. |
| Brand, model offered | Brand name (for identification purposes) and model name (for identification purposes). |
| Purpose | Combi steamer must function as a core cooking centre in a CPU (central production unit), suitable for proteins, vegetables, mixed dishes, casseroles and steam-based production cycles. Must support convection, steam and combi modes with automatic transitions. |
| Capacity (GN loading, per cycle) | Unit must accept a full roll-in trolley loaded with 36–40 GN 1/1 trays per full loading cycle, depending on trolley configuration. THE PARTICIPANT shall specify the exact GN 1/1 tray quantity per trolley offered. |
| Chamber compatibility | Cooking chamber must be designed for 20 shelf levels per trolley, GN1/1 or EN 600×400 compatible. |
| Overall external dimensions (L × W × H) | Dimensions must fall within: |
|  | • length 1250–1350 mm, |
|  | • width 1150–1250 mm, |
|  | • height 2000–2200 mm. |
| Chamber materials and structure | Cooking chamber from stainless steel minimum AISI 304, rounded corners, reinforced hinges, high-temperature gaskets, hygienic welds. |
| Door | Insulated, double-glazed door with safe ventilation and door micro-switch stopping fan operation when open. |
| Heating and steam system | Combi steamer must use a combined steam generation principle (direct injection and/or steam-generator hybrid), ensuring reliable steam output at low and high humidity loads. Supplier must describe the steam generation method. |
| Humidity management | Automatic humidity control system with humidity compensation to stabilise chamber environment during door openings and high-load steaming. |
| Temperature range | Operating temperature 30°C to 300°C (or equivalent manufacturer range). |
| Core probe | Unit must include multi-point or heated core probe integrated with cycle programming. |
| Air circulation system | High-efficiency fan system ensuring even heat distribution across all GN levels. Multi-speed or reversible airflow acceptable. |
| Programmable cycles | Must support convection, steam, combi, low-temperature cooking, regeneration, overnight cooking, automatic cycles and custom user programs. |
| Cleaning system | Integrated automatic cleaning system (liquid or solid detergents), with self-cleaning and rinse programs. |
| Control panel | Digital control interface with temperature settings, humidity % settings, fan modes, cycle programming, probe management, alarms and monitoring. |
| Power supply | 400V 3~ 50Hz (or 400V 3+N 50Hz). Total electrical load must be indicated. |
| Safety features | Door micro-switch, thermal overload protection, emergency stop, safe chamber ventilation, compliance with EU industrial safety directives. |
| Installation | Unit must be delivered fully assembled, requiring only electrical and drain/exhaust connection. No chamber assembly allowed at customer site. |
| Complete functional set (mandatory) | The following must be included in the offered price: |
|  | • roll-in combi steamer unit |
|  | • full GN1/1 trolley (36–40 trays capacity) |
|  | • rack alignment/docking system |
|  | • multi-point or heated core probe |
|  | • automatic cleaning system |
|  | • all rails, guides, filters or platform components required for operation |
|  | • condensate drain connections |
| THE PARTICIPANT must provide a complete list of all included parts. |  |
| Documentation | Must be supplied in Latvian or English: user manual, installation manual, technical passport, maintenance instructions, electrical diagrams, declaration of conformity. |
| Compliance and CE marking | Device must comply with EU Machinery Directive, EMC and LVD requirements. CE marking mandatory. |
| Guarantee | Minimum 24 months full manufacturer or supplier warranty. |

Name, surname, position

signature