# Annex No. 2

TECHNICAL SPECIFICATION

PURCHASE OF SHOCK FREEZER CHAMBER (TROLLEY TYPE)

place, date

| Parameter | Requirement |
| --- | --- |
| THE PARTICIPANT shall provide information or proof against each requirement. |  |
| Quantity | 2 pcs. – Acoustic shock-freezing chambers for deep-freezing of trolley-loaded food products. |
| Brand, model offered | Brand name (for identification purposes) and model name (for identification purposes). |
| Purpose | Chamber must be designed for deep shock freezing of bakery, pastry, RTE and par-baked products using full GN trolley loads, with intensive airflow and acoustic (or equivalent energy-based) freezing enhancement. |
| Freezing performance | Chamber must freeze 1000–1500 kg per cycle, reaching product core temperature –18°C within a 6–8 h deep-freezing cycle at chamber air temperature –40°C. Supplier must confirm certified performance. |
| Temperature range | Operating range must allow cycles from +90°C down to –40°C. |
| Overall dimensions (L × W × H) | Dimensions must fall within: |
|  | • length 3500–3700 mm |
|  | • width 2900–3100 mm |
|  | • height 2600–2800 mm |
| Internal chamber | Internal dimensions must allow insertion of full GN1/1, GN2/1 or 600×400 bakery trolleys. Rounded internal corners required. Supplier must specify internal dimensions. |
| Chamber construction | Monoblock design from stainless steel minimum AISI 304, with scotch-brite or satin finish. |
| Insulation | Insulation thickness must be not less than 100 mm, with high-density (approx. 40 kg/m³) polyurethane foam. |
| Airflow system | Must include forced-air system with multi-vortex (minimum 4-vortex) distribution ensuring uniform freezing across all trolley positions. |
| Acoustic freezing technology | Chamber must include acoustic, ultrasonic, or equivalent energy-based freezing intensification technology to enhance nucleation and freezing uniformity. Supplier must describe the method. |
| Trolley type compatibility | Chamber must accept GN1/1, GN2/1 or 600×400 bakery trolley systems. Supplier shall state trolley compatibility. |
| Standard equipment | Must include: |
|  | • heated core probe |
|  | • removable magnetic door gasket |
|  | • micro-switch for fan stop on door open |
|  | • stainless-steel adjustable feet or floor rails |
|  | • accessible evaporator area |
|  | • internal lighting |
| Defrost system | Automatic hot-gas or equivalent high-efficiency defrost system. |
| Refrigeration system | Remote condensing unit (air-cooled or water-cooled). Capacity must be sufficient to maintain –40°C chamber temperature under full load. |
| Refrigerant | Acceptable refrigerants: R452A, R290, or equivalent EU-compliant low-GWP refrigerant. |
| Power supply | 400V 3~ or 400V 3+N 50Hz. Total installed power approx. 18 kW must be indicated. |
| Hygiene and cleaning | Chamber must allow full cleaning access; evaporator must be easily accessible. Drain system must be integrated. |
| Safety features | Emergency stop, thermal protections, fan stop on open door, compliance with EU industrial safety requirements. |
| HACCP logging | Integrated HACCP logging with time/temperature recording and USB export (or equivalent). |
| Installation | Chamber must be delivered fully assembled, requiring only electrical, drain and refrigeration connection to the remote condensing unit. No structural assembly permitted on site. |
| Complete functional set (mandatory) | Offer must include: |
|  | • shock-freezing chamber |
|  | • remote condensing unit |
|  | • heated core probe |
|  | • door with magnetic gasket |
|  | • evaporator & fan group |
|  | • airflow vortex system |
|  | • trolley access system (floor rails or compatible feet) |
|  | • any accessories necessary for full operation |
| THE PARTICIPANT must provide a complete list of all included components. |  |
| Documentation | To be supplied in Latvian or English: user manual, installation manual, technical passport, HACCP instructions, maintenance instructions, electrical diagrams, declaration of conformity. |
| Compliance and CE marking | Must comply with EU Machinery Directive, EMC, LVD and refrigerant legislation. CE marking mandatory. |
| Guarantee | Minimum 24 months full manufacturer or supplier warranty. |

Name, surname, position

signature