Annex No. 2

TECHNICAL SPECIFICATION

PURCHASE OF SPIRAL MIXER WITH REMOVABLE BOWL

place, date

The following table:

| Parameter | Requirement |
| --- | --- |
| THE PARTICIPANT shall provide information or proof against each requirement. |  |
| Quantity | 1 pc. – Professional spiral mixer with lifting head and removable bowl. |
| Brand, model offered | Brand name (for identification purposes) and model name (for identification purposes). |
| Purpose | Kneading of pizza dough and small quantities of bread dough. The machine must feature a manually liftable head and a removable bowl to facilitate product removal and cleaning operations. |
| Capacity | • Dough Capacity: 30 – 40 kg. • Flour Capacity: 18 – 22 kg. • Bowl Volume: 45 – 50 litres. |
| Construction & Materials | • Bowl, Spiral, Shaft: High-resistance Stainless Steel. • Breaking Bar: Must feature a central stainless steel column/bar to improve kneading performance. • Lid: PETG (transparent plastic) or equivalent safety cover with a hole/opening for adding ingredients during operation. • Structure: Painted steel or equivalent hygienic finish. |
| Mechanism | • Head Lifting: Manual opening/raising of the mixer head via pneumatic piston or spring assistance. • Bowl Removal: Manual mechanism to extract the bowl. • Transmission: Mechanical reducer and chain drive for reliability. • Moving Parts: Kinematic motion on pre-lubricated ball bearings (maintenance-free). |
| Motor & Power | • Power: 1.0 kW – 1.5 kW. • Voltage: 400V / 3 ph / 50 Hz. |
| Dimensions & Weight | • Width: 460 – 500 mm. • Depth: 800 – 850 mm. • Height: 780 – 830 mm. • Net Weight: 110 – 130 kg. |
| Bowl Dimensions | • Diameter: 440 – 460 mm. • Height: 290 – 310 mm. |
| Complete Functional Set (Mandatory) | The supplier is responsible for providing a fully functional unit. The offer must include: • Mixer body with lifting head mechanism • 1 x Removable Stainless Steel Bowl • Spiral tool and breaking column • Safety lid • Wheels for movement (if standard) or adjustable feet • Power cable |
| Documentation | To be supplied in Latvian or English: user manual, electric diagram, maintenance instructions, CE Declaration of Conformity. |
| Compliance | Equipment must comply with EU Machinery Directive 2006/42/EC. CE marking mandatory. |
| Guarantee | Minimum 24 months full manufacturer or supplier warranty. |

Name, surname, position

signature