Annex No. 2

TECHNICAL SPECIFICATION

PURCHASE OF STAINLESS STEEL VEGETABLE PREPARATION TABLE

place, date

The following table:

| Parameter | Requirement |
| --- | --- |
| THE PARTICIPANT shall provide information or proof against each requirement. |  |
| Quantity | 4 pcs. – Professional stainless steel preparation and washing table specifically designed for vegetables. |
| Brand, model offered | Brand name (for identification purposes) and model name (for identification purposes). |
| Purpose | Washing, peeling, and cutting of vegetables in a commercial kitchen or food processing facility. |
| Construction | • Material: High-quality Stainless Steel AISI 304 (18/10) throughout (top, legs, shelf, sink) to ensure maximum hygiene and durability. • Top: Reinforced worktop with sound-deadening underlay, featuring a special anti-drip edge (marine edge) or raised rim to contain water and vegetable scraps. |
| Sink & Work Area | • Configuration: Integrated sink bowl welded into the worktop (Standard configuration: Sink on the Right or Left side - to be specified or consistent with TPL1870D which usually implies a specific side, e.g., Right). • Sink Dimensions: min. 400 x 400 x 250 mm (or similar suitable for washing vegetables). • Work Surface: Large flat preparation area adjacent to the sink. • Waste Hole: Includes a scraped waste hole (approx. Ø 180-220 mm) with a rubber ring for easy disposal of peels directly into a bin underneath. |
| Structure & Legs | • Legs: Square or round stainless steel legs (e.g., 40x40 mm). • Feet: Adjustable stainless steel feet to level the table on uneven floors. • Shelf: Solid or perforated stainless steel lower shelf for storage (optional but standard on this model). |
| Dimensions | • Length: 1750 mm – 1850 mm. • Depth: 650 mm – 750 mm. • Height: 850 mm – 900 mm (Adjustable). |
| Plumbing | • Drain: Standard drain fitting with overflow pipe. • Tap: Hole for mixer tap (or included mixer tap if specified). |
| Complete Functional Set (Mandatory) | The supplier is responsible for providing a fully functional unit ready for assembly and use. The set must include: • Stainless steel table structure • Integrated waste hole with rubber guard • Drain siphon/waste kit • Mixer Tap: Lever-operated mixer tap with shower spray (pre-rinse unit) or standard high-neck tap suitable for vegetable washing. |
| Documentation | To be supplied in Latvian or English: assembly instructions, maintenance guide, Declaration of conformity (food contact materials). |
| Compliance | Equipment must comply with EU Regulation EC 1935/2004 (materials in contact with food) and HACCP hygiene standards. |
| Guarantee | Minimum 24 months full manufacturer or supplier warranty. |

Name, surname, position

signature