## Annex No. 2

## Technical Specification

### Purchase of Compact Industrial Fish Scalers

| Parameter | Requirement |
| --- | --- |
| THE PARTICIPANT shall provide information or proof against each requirement. |  |
| Quantity | 2 pcs. – Compact Industrial Electric Fish Scalers (Tabletop/Benchtop unit). |
| Purpose | Efficient, fast, and hygienic removal of scales from various types of fish. Designed for continuous use in fish processing facilities, maximizing yield while minimizing product damage. |
| Model Class / Operation |  |
| * Reference Model | The technical baseline is set by the functionality and parameters of the Fish Scaler KT-S class of equipment. |
| * Scaler Type | Hand-held rotary scaler head attached to a flexible shaft (removable/interchangeable). |
| * Scaler Head | Must use rotary/circular scaling heads with protective cover. |
| Technical & Energy |  |
| * Power / Voltage | Must be electrically powered. Single-phase industrial operation recommended. |
| * Scaler Motor** | The motor unit (power unit) must be compact and mounted on a base suitable for tabletop/benchtop use. |
| * RPM | Output speed of the scaler head should be optimized for effective scale removal. |
| Construction & Hygiene |  |
| * Material | Housing and all components in contact with the process must be constructed from Stainless Steel (AISI 304 or equivalent) or robust, non-corrosive, food-grade materials. |
| * Safety | Includes protection against electric shock and accidental engagement of the flexible shaft. |
| Mandatory Accessories (Complete Set) | The price must include all accessories required for immediate and continuous operation, specifically: |
|  | 1. Flexible Shaft: Minimum of two units per scaler machine (total 4). |
|  | 2. Scaling Heads: A complete set of different sizes/types of scaling heads (cutters) for various fish species (e.g., small, medium, large fish). |
|  | 3. Hanging Bracket/Mount: A secure bracket or hook for mounting the motor unit near the workstation (optional use). |
|  | 4. Tools: Essential tools for changing heads and maintenance. |
| Complete Functional Set (Mandatory) | The Supplier must fully complete the equipment and is responsible for confirming with the procurement manager that all necessary components are included for the units to be fully functional and capable of fulfilling their assigned tasks. The offer must explicitly include: |
|  | • The main motor unit and housing. |
|  | • All mandatory accessories listed above. |
|  | • Installation materials, utility connections, commissioning, and comprehensive staff training. |
| Documentation | To be supplied in Latvian or English: user manual, electric diagram, maintenance instructions, and CE Declaration of Conformity. |
| Guarantee | Minimum 24 months full manufacturer or supplier warranty. |