## Annex No. 2

## Technical Specification

### Purchase of Custom Industrial Sterile Lid Drying Rack

| Parameter | Requirement |
| --- | --- |
| THE PARTICIPANT shall provide information or proof against each requirement. |  |
| Quantity | 1 pc. – Custom-Designed Sterile Lid Drying Rack . |
| Intended Use (Purpose) | Hygienic drying and temporary storage of disinfected twist-off lids (sized for 50g – 200g caviar jars) immediately after hot-water disinfection (70C - 80C). Must ensure rapid drainage and efficient air drying. |
| 1. Construction and Materials (Critical) |  |
| * Material (Mandatory) | AISI 316 stainless steel for the entire construction (frame, shelves, supports, fasteners). This is mandatory due to the corrosive saltwater/seafood environment. |
| * Welding & Finish | Fully TIG welded, smooth, continuous, fully cleaned, and passivated. Surface finish: Ground / satin, 0.8 μm. |
| * Hygiene Design | All corners rounded (R3), with no sharp edges, crevices, or "dead pockets". |
| 2. Dimensions and Capacity |  |
| * Overall Dimensions (L x W x H) | 800 mm – 820 mm (L) x 490 mm – 510 mm (W) x 1390 mm – 1410 mm (H) (10 mm tolerance). |
| * Shelf Quantity | Minimum 3 perforated drying shelves, 4 shelves recommended. |
| * Total Capacity | Total rack capacity: 300 – 500 lids per cycle. |
| 3. Shelves (Perforated Drying Levels) |  |
| * Material / Perforation | AISI 316 perforated sheet with 4 – 5 mm and 30% - 40% open area. |
| * Function | Must enable rapid drainage, uniform air drying, and prevention of water pooling. |
| * Load / Access | Load rating 12 kg per shelf. Shelves must be fully removable for cleaning and sanitation. |
| 4. Mobility and Stability |  |
| * Wheels (Mandatory) | 4 industrial food-grade wheels min. 125 mm, all stainless-steel housings, with at least 2 wheels featuring total-stop brakes. |
| * Stability | Structural frame must ensure zero wobble under full load. |
| 5. Optional/Recommended Features (Must be quoted separately) |  |
| * Protective Cover | Transparent hygienic cover (hood, made from food-grade PC or transparent PVC curtain) to protect drying lids from dust during cooling & handling. |
| * Drainage | Drainage channels recommended under the lowest shelf. |
| * Identification | Laser-etched identification plate (Manufacturer, Model: SLDR-500, Year, Batch/Serial, Material: AISI 316). |
| 6. Compliance and Warranty |  |
| * Standards | Design must comply with HACCP, ISO 22000, and EHEDG good hygiene practice. |
| * Marking | CE marking is mandatory. |
| Complete Functional Set (Mandatory) | The Supplier must ensure the delivery of a single fully functional mobile rack, including 3 – 4 removable perforated AISI 316 shelves and the 4 wheels (2 locking). The Supplier must contact the Procurement Manager for final confirmation of parameters not fully specified by the technical task. |
| Documentation | Technical documentation (datasheet, materials certificates, cleaning instructions, spare part list) and CE Declaration of Conformity must be supplied. |
| Guarantee | Minimum 24 months full manufacturer or supplier warranty. |