## Annex No. 2

## Technical Specification

### Purchase of Industrial Blast Chiller / Shock Freezer (Air-Condensed)

| Parameter | Requirement |
| --- | --- |
| THE PARTICIPANT shall provide information or proof against each requirement. |  |
| Quantity | 1 pc. – Industrial Blast Chiller / Shock Freezer Cabinet. |
| Purpose | To perform rapid chilling and shock freezing with proving and defrosting functions of cooked food products within HACCP time limits, ensuring food safety, extending shelf life, and minimizing product dehydration. Suitable for professional catering and industrial kitchens. |
| 1. Capacity and Dimensions (10 Tray Class) |  |
| * Capacity Class | Must accommodate 16 – 20 trays (GN 1/1 or 600x400 mm) of equivalent capacity. |
| * Internal Configuration (Critical) | The interior cabinet dimensions must be correctly sized to allow a compatible roll-in/roll-out trolley to be placed directly into the chilling chamber. |
| * Chilling Capacity (Output) | Min. capacity: 25 kg – 35 kg (from +90C to +3C in< 90 min). |
| * Freezing Capacity (Output) | Min. capacity: 18 kg – 22 kg (from +90C to -18C in < 240 min). |
| * External Width (W) | 790 mm – 890 mm. |
| * External Depth (D) | 840 mm – 1320 mm. |
| * External Height (H) | 1650 mm – 2420 mm. |
| 2. Cooling System (Critical) |  |
| * Condenser Type | Must be supplied with an integrated Air-Cooled (Air-Condensed) Condenser Unit (Self-Contained Unit). |
| * Cooling System | Forced air circulation (ventilated) for uniform temperature distribution. |
| * Refrigerant Type | Environmentally friendly hydrocarbon refrigerant (e.g., R290, R452A) or equivalent low GWP refrigerant. |
| * Defrost | Automatic hot gas defrosting system. |
| 3. Construction and Features |  |
| * Material | AISI 304 Stainless Steel monobloc structure. |
| * Insulation | High-density HFC-free polyurethane foam insulation (minimum 70 mm thickness). |
| * Interior Design | Chambers must have rounded internal edges for easier cleaning and hygiene. |
| * Core Probe | Must include a heated core probe as a standard feature. |
| * Controls | Digital electronic control panel for precise temperature monitoring, control, and automatic program execution. |
| 4. Technical Requirements |  |
| * Supply Voltage | 400V, 3 Phase, 50 Hz (or compatible industrial voltage). |
| Complete Functional Set (Mandatory) | The Supplier must ensure the delivery of a single fully functional Blast Chiller unit, including the required air-cooled condenser, core probe, and all internal racks/guides. The offer must explicitly include: installation materials, final utility connections, commissioning, and comprehensive staff training. |
| Documentation | To be supplied in Latvian or English: user manual, electric diagram, maintenance instructions, and CE Declaration of Conformity. |
| Guarantee | Minimum 24 months full manufacturer or supplier warranty. |