## Annex No. 2

## Technical Specification

### Purchase of Industrial Column Lifting and Tipping Devices (EuroBin Lifters)

| Parameter | Requirement |
| --- | --- |
| THE PARTICIPANT shall provide information or proof against each requirement. |  |
| Quantity | 2 pcs. – Industrial Column/Mast Lifting and Tipping Devices (Vertical Lifting Devices). |
| Purpose | Vertical lifting and controlled tipping of standard 200-liter industrial containers (Eurobins, DIN 9797) into subsequent processing machines (mixers, grinders, etc.), optimizing ergonomic conditions and production flow. |
| Operational Specifications |  |
| * Maximum Lifting Load | 280 kg – 320 kg. |
| * Container Compatibility | Designed specifically for lifting and tipping standard 200 L Eurobins (DIN 9797). |
| * Lifting Height (Tipping Edge) | Working tipping height range: 1800 mm – 2200 mm (To be confirmed by Procuring Authority based on receiving machine height). |
| * Mechanism | Lifting mechanism must ensure gentle and controlled tipping. |
| Construction & Hygiene |  |
| * Material | Hygienic, robust construction made entirely of Stainless Steel (AISI 304 or equivalent). |
| * Column Design | Column (mast) design must be open and hygienic, allowing dirt and water to fall away directly during cleaning. |
| * Safety Features | Integrated and secured control system/buttons. Cables must run inside the column for maximum safety and tidiness. |
| * Base | Quick levelling ensured by adjustable machine feet. |
| Technical & Energy |  |
| * Electrical Supply | 400 V, 3 Phase, 50 Hz (or compatible industrial voltage). |
| * Control | Simple controls (e.g., "Up," "Down," "Emergency Stop"). |
| Complete Functional Set (Mandatory) | The Supplier must ensure the delivery of two fully functional units, complete for industrial lifting and tipping. The Supplier must ensure that the equipment, including the necessary attachments and final tipping height adjustments, fully performs its assigned tasks and must coordinate with the procurement manager on the required tipping height. The offer must explicitly include: installation materials, commissioning, and comprehensive staff training. |
| Documentation | To be supplied in Latvian or English: user manual, electric diagram, maintenance instructions, and CE Declaration of Conformity. |
| Guarantee | Minimum 24 months full manufacturer or supplier warranty. |