## Annex No. 2

## Technical Specification

### Purchase of Industrial Double Sink Units with Legs

| Parameter | Requirement |
| --- | --- |
| THE PARTICIPANT shall provide information or proof against each requirement. |  |
| Quantity | 15 pcs. – Industrial Double Sink Units (Sinks on Legs, 700 Series Depth). |
| Purpose | High-volume manual washing of hands and utensils in commercial and industrial kitchens, complying with the highest hygiene standards. |
| 1. External Dimensions |  |
| * External Length (L) | 1380 mm – 1420 mm. |
| * External Depth (D) | 690 mm – 710 mm. |
| * External Height (H) | 840 mm – 860 mm (Working surface height). |
| 2. Sink Bowl Configuration |  |
| * Bowl Quantity | Two (2) deep sink bowls. |
| * Bowl Dimensions (L x W x H) | Each bowl must have dimensions in the range: 580 mm} - 610 mm (L); 490 mm - 510 mm (W); 290 mm - 310 mm (H). |
| * Bowl Capacity | The bowls must be sufficiently deep (min. 300 mm range) for industrial washing. |
| 3. Construction and Worktop |  |
| * Material | Manufactured from AISI 304 Stainless Steel. |
| * Worktop Edge | Worktop must have a flat edge thickness of 40 mm. |
| * Splashback | Includes a rear upstand (splashback) of 100 mm height. |
| * Assembly | Units must be supplied disassembled (for volume reduction) but designed for rapid assembly. |
| * Legs/Base | Must include adjustable feet/legs (not castors) for stable floor mounting. |
| 4. Plumbing and Installation |  |
| * Waste Outlets | Each of the two bowls must be equipped with its own waste outlet (DN40 or DN50 standard). |
| * Taps | Standard tap hole mounting area on the splashback/rear upstand. |
| Complete Functional Set (Mandatory) | The Supplier must ensure the delivery of 15 complete and fully functional sink units. The offer must explicitly include: the sink unit (bowls/worktop/splashback), legs, adjustable feet, and all necessary assembly hardware. |
| Documentation | To be supplied in Latvian or English: user manual, assembly instructions, and CE Declaration of Conformity. |
| Guarantee | Minimum 24 months full manufacturer or supplier warranty. |