## Annex No. 2

## Technical Specification

### Purchase of Industrial Double-Chamber Vacuum Packaging Machine

| Parameter | Requirement |
| --- | --- |
| THE PARTICIPANT shall provide information or proof against each requirement. |  |
| Quantity | 1 pc. – Industrial Double-Chamber Vacuum Packaging Machine. |
| Purpose | High-volume, reliable vacuum packaging of food products to extend shelf life, prevent freezer burn, and ensure hygienic storage, suitable for continuous industrial operation. |
| 1. Dimensions and Capacity (Polar 2-40 Class) |  |
| * Chamber Dimensions (L x W x H) | 400 mm – 410 mm (L) x 620 mm – 630 mm (W) x 240 mm – 250 mm (H). |
| * Machine Dimensions (L x W x H) | 810 mm – 830 mm (L) x 1480 mm – 1500 mm (W) x 1340 mm – 1360 mm (H). |
| * Pump Capacity (50 Hz) | 60 m³/h – 65 m³/h. |
| * Machine Cycle Time (50 Hz) | Fast cycle time: 12 seconds – 35 seconds. |
| * Sealing Bars | Supplied with two sealing bars as standard (front/rear). |
| 2. Construction and Hygiene |  |
| * Construction Material | Stainless steel casing and flat working plate. |
| * Safety/Hygiene Rating | Minimum IP-65 classification. |
| * Cleaning | Easy-to-remove silicone holders for easy cleaning. |
| 3. Technical Requirements |  |
| * Supply Voltage | Must be operable at 400V, 3-Phase, 50 Hz. |
| * Power | Power consumption: 2.0 kW – 3.0 kW. |
| 4. Mandatory Features and Accessories |  |
| * Control System (Required) | Advanced Control System (ACS). |
| * Sealing Type (Required) | Double seal as standard (two sealing wires). |
| * Gas Flush (Required) | Gas flush option (including necessary compressed air connection). |
| * Sensor Control (Required) | Sensor control is mandatory. |
| * Liquid Control (Required) | Liquid control feature is mandatory. |
| * Printer (Required) | Thermal label printer (must be compatible with ACS). |
| * Inclined Plate (Required) | Inclined insert plate (for packaging liquids and powders). |
| * Maintenance Set (Required) | Maintenance set for standard upkeep. |
| Complete Functional Set (Mandatory) | The Supplier must ensure that all components and preparation of the machine are completed for full operation. If data is lacking, the supplier must contact the procurement department. The offer must explicitly include: the machine, vacuum pump, all mandatory accessories listed in Section 4, and four insert plates. |
| Documentation | To be supplied in Latvian or English: user manual, electric diagram, maintenance instructions, and CE Declaration of Conformity. |
| Guarantee | Minimum 24 months full manufacturer or supplier warranty. |