## Annex No. 2

## Technical Specification

### Purchase of Industrial Electric Bain-Marie Module

| Parameter | Requirement |
| --- | --- |
| THE PARTICIPANT shall provide information or proof against each requirement. |  |
| Quantity | 1 pc. – Industrial Electric Bain-Marie Module (Open Base or equivalent). |
| Purpose | Designed to maintain food at the correct temperature, or for delicate processes like melting chocolate or preparing creams, ensuring professional quality and hygiene. Suitable for seamless modular integration with other 700 series cooking equipment. |
| 1. External Dimensions |  |
| * External Width (W) | 390 mm – 410 mm (Standard single module width). |
| * External Depth (D) | 710 mm – 730 mm (Standard 700 Series depth). |
| * External Height (H) | 880 mm – 900 mm (Working height for modules with open base). |
| 2. Technical Specifications |  |
| * GN Capacity | Designed to accommodate 1 x GN 1/1 pan. |
| * Heating | Heating via silicon-coated, plated heating elements fastened under the tank bottom or via silicone heating pad external to the tank. |
| * Temperature Control | Temperature regulation via thermostat with a range of 30C to 90C. |
| * Water Fill | Manual water fill via a filling nozzle. |
| * Drainage | Water drainage controlled by a solenoid valve. |
| * Power Load | Electrical connected load: 1.0 kW – 1.3 kW. |
| * Power Supply | Voltage: 230V or 400V (to be specified by client). Frequency: 50 Hz. |
| 3. Construction and Hygiene |  |
| * Material | Robust construction from AISI 304 grade stainless steel. |
| * Tank Design | Tank bottom must be tilted/sloped to improve water flow toward the drain. Tank interior must have rounded corners for improved cleaning. |
| * Base | Supplied as a module on an open base cabinet with height-adjustable stainless steel feet. |
| * Maintenance | Heating elements placed externally to the tank (e.g., silicone pad) for superior hygiene and ease of cleaning. |
| Complete Functional Set (Mandatory) | The Supplier must ensure the delivery of a fully functional Bain-Marie unit, including the required heating elements, thermostat, solenoid valve for draining, and indicator lights. The offer must explicitly include: installation materials, commissioning, and staff training. |
| Documentation | To be supplied in Latvian or English: user manual, electric diagram, maintenance instructions, and CE Declaration of Conformity. |
| Guarantee | Minimum 24 months full manufacturer or supplier warranty. |