## Annex No. 2

## Technical Specification

### Purchase of Industrial Fish Processing Tables (Sorting/Preparation)

| Parameter | Requirement |
| --- | --- |
| THE PARTICIPANT shall provide information or proof against each requirement. |  |
| Quantity | 4 pcs. – Industrial Fish Processing Tables (Sorting/Preparation Tables). |
| Purpose | Manual sorting, preparation, portioning, and trimming of fish products. The design must be extremely hygienic and resistant to the corrosive environment of fish processing (salt/brine/water). |
| 1. External Dimensions |  |
| * External Length (L) | 1580 mm – 1620 mm. |
| * External Depth (D) | 690 mm – 710 mm. |
| * External Height (H) | 840 mm – 860 mm. |
| 2. Construction and Material (Critical) |  |
| * Material (Minimum) | Manufactured from AISI 304 Stainless Steel. |
| * Material (Recommendation) | Due to the corrosive environment of fish processing, the material should be specialized or reinforced (AISI 316 is highly recommended). |
| * Typology | Designed with legs/frame for free-standing floor mounting. |
| 3. Worktop Configuration (Custom Layout) |  |
| * Worktop Thickness | Steel worktop surface must have a thickness equivalent to 38 mm – 42 mm. |
| * Worktop Edge | The worktop edge (worktop thickness visible from the side) must be at least 40 mm. |
| * Splashback | Must include a rear upstand (splashback) with a height of 90 mm – 110 mm. |
| * Waste Disposal | An aperture/chute must be integrated on the right side of the worktop for quick fish waste disposal. |
| * Cutting/Processing Zone | A specialized food-grade plastic insert/strip (PE or equivalent) must be fitted on the left side of the worktop for cutting/processing, with a maximum width of 180 mm – 240 mm. |
| * Drainage Zone | The remaining stainless steel work surface must be fabricated with a slight depression/recess to facilitate liquid drainage/run-off towards the rear. |
| * Hygiene | All surfaces must be smooth, fully welded, and designed for easy cleaning and high-pressure washing. |
| * Legs/Base | Must include adjustable stainless steel feet/legs for stable floor mounting. |
| 4. Compliance |  |
| * Standards | Must comply with applicable EU food contact material regulations and hygiene standards (HACCP). |
| Complete Functional Set (Mandatory) | The Supplier must ensure the delivery of four complete and fully functional table units, incorporating the specified custom layout (left-side plastic strip, right-side waste chute, recessed drainage surface). The offer must explicitly include: the table unit (worktop/splashback/frame), legs, adjustable feet, and the fitted plastic cutting strip. |
| Documentation | To be supplied in Latvian or English: user manual, assembly instructions, and CE Declaration of Conformity. |
| Guarantee | Minimum 24 months full manufacturer or supplier warranty. |