## Annex No. 2

## Technical Specification

### Purchase of Industrial Polymer Tub Trolleys

| Parameter | Requirement |
| --- | --- |
| THE PARTICIPANT shall provide information or proof against each requirement. |  |
| Quantity | 10 pcs. – Industrial Polymer Tub Trolleys (Tub Trucks). |
| Purpose | Hygienic and ergonomic handling, transport, and temporary storage of food products, ingredients, or waste materials within a food processing environment. |
| Tub Specifications |  |
| * Nominal Volume | 190 Litres – 210 Litres (Standard industrial capacity). |
| * Tub Material | Manufactured from food-grade, seamless High-Density Polyethylene (HDPE) or equivalent robust industrial polymer. |
| * Construction | Seamless, one-piece construction with smooth internal surfaces for maximum hygiene and easy cleaning. |
| Chassis & Mobility |  |
| * Chassis Material | Robust undercarriage/chassis frame made from Stainless Steel (AISI 304 or equivalent). |
| * Wheels | Equipped with 4 heavy-duty swivel castors (wheels) suitable for continuous industrial use and wet environments. |
| External Dimensions |  |
| * Length (L) | 780 mm – 820 mm. |
| * Width (W) | 580 mm – 620 mm. |
| * Height (H) | 680 mm – 720 mm (Total height including wheels). |
| Mandatory Accessories (Required for Task) | The price must include the following accessories, which are crucial for maintaining hygiene during food handling: |
|  | 1. Polymer Lid: A secure, tight-fitting food-grade polymer lid for every trolley (total 10 pcs.) to protect contents during transport and storage. |
|  | 2. Drain Plug/Valve: An integrated drain plug or valve at the lowest point of the tub for fast and hygienic draining and cleaning. |
| Complete Functional Set (Mandatory) | The Supplier must ensure the delivery of ten fully functional tub trolleys, complete with the specified accessories, ensuring they are suitable for high-hygiene food processing tasks. The offer must explicitly include: the tub, the stainless steel chassis, the four wheels, the lid, and the drain plug/valve. |
| Documentation | To be supplied in Latvian or English: maintenance instructions and CE Declaration of Conformity. |
| Guarantee | Minimum 24 months full manufacturer or supplier warranty. |