## Annex No. 2

## Technical Specification

### Purchase of Industrial Roe Separator

| Parameter | Requirement |
| --- | --- |
| THE PARTICIPANT shall provide information or proof against each requirement. |  |
| Quantity | 1 pc. – Industrial Roe Separator. |
| Purpose | For the efficient and hygienic separation of fish roe from membranes and other unwanted parts (Roe/Skein separation). Designed for gentle and consistent roe processing in a fish processing environment. |
| External Dimensions |  |
| * External Length (L) | 1200 mm – 1650 mm. |
| * External Width (W) | 700 mm – 900 mm. |
| * External Height (H) | 1200 mm – 1550 mm. |
| Operational Performance |  |
| * Throughput Capacity | The machine's throughput capacity must be 30 kg/h – 80 kg/h (depending on the fish species and operator efficiency). |
| * Separation Principle | Operation based on a mechanical drum designed for gentle roe separation, without altering its structure. |
| * Drum Perforation | The separating drum perforation diameter must be 2.0 mm – 3.0 mm. |
| Construction and Hygiene |  |
| * Main Construction Material | The machine frame and all parts in contact with the product must be made of Stainless Steel AISI 304 (or equivalent food-grade material). |
| * Corrosion Resistance (Option) | AISI 316 Stainless Steel (for higher resistance to salt and acids) is preferred / mandatory according to the buyer's requirement (the proposal must state the capability). |
| * Cleaning | Must feature an easily cleanable design with straightforward access to the drum and internal components. |
| Water and Drainage System |  |
| * Water Supply | Integrated spray bar mechanism for drum surface cleaning. Supply must be adapted for low-pressure cold water line connection. |
| * Drainage Outlet | Liquid drainage outlet diameter must be 40 mm – 50 mm, adapted for connection to an industrial trough or drainage line. |
| Technical Requirements |  |
| * Motor Power | Nominal motor power: 0.35 kW – 0.80 kW. |
| * Electrical Supply | 3x400V, 50 Hz (or equivalent three-phase industrial voltage). |
| Complete Functional Set (Mandatory) | The Supplier must fully complete the equipment, taking into account its intended purpose (effective roe separation). The offer must explicitly include: |
|  | • The complete separator unit with drum, motor, and frame. |
|  | • All specified water spray and drainage components. |
|  | • Installation materials and final utility connections. |
|  | • Commissioning and comprehensive staff training. |
| Documentation | To be supplied in Latvian or English: user manual, electric diagram, maintenance instructions, and CE Declaration of Conformity. |
| Guarantee | Minimum 24 months full manufacturer or supplier warranty. |