## Annex No. 2

## Technical Specification

### Purchase of Industrial Two-Station Active Trimming Lines (ATL)

| Parameter | Requirement |
| --- | --- |
| THE PARTICIPANT shall provide information or proof against each requirement. |  |
| Quantity | 4 pcs. – Industrial Active Trimming Lines (ATL), configured for simultaneous use by two (2) operators. |
| Purpose | High-efficiency trimming, portioning, deboning (pin bone checking), and quality control (skin checking) of fish fillets or similar food products. The system must minimize water and product waste on the floor. |
| Operator Stations | The line must be configured for exactly 2 individual operator stations. |
| Conveyor System (Mandatory) |  |
| * Main Conveyor | Must include a central conveyor for continuous transport of fillets/main product along the processing stations. |
| * Waste Conveyor | Must include a secondary, integrated waste conveyor (or chute system leading to an integrated waste conveyor platform) for immediate removal of trimming waste. |
| Design & Construction |  |
| * Material | Entire line must be constructed from Stainless Steel (AISI 304 or equivalent). |
| * Design Type (Base) | Standard, robust design or Tube Design (recommended for waste and water reduction). |
| * Cleaning | Must feature an easy-to-clean design, with easy access to internal components (e.g., elevating system for Tube Design). |
| * Individual Work Area | The line must be designed to accommodate individual work surfaces or individual trimming tables for each operator (especially recommended if production capacity varies). |
| * Ergonomics | Working height must be easily adjustable (via adjustable feet or platform system) for optimal operator comfort. |
| Approximate Dimensions (Subject to Final Layout Confirmation) |  |
| * Total Length (L) | 2000 mm – 2500 mm (Length required for 2 stations plus end sections). |
| * Total Width (W) | 1000 mm – 1200 mm (Width accommodating conveyor, operator space, and waste chute). |
| * Working Height (H) | 900 mm – 1100 mm (Adjustable standard working height). |
| Technical & Energy |  |
| * Power / Voltage | 3x400V, 50 Hz (or compatible industrial voltage). |
| * Control | Variable speed control capability for the main conveyor belt. |
| Complete Functional Set (Mandatory) | The Supplier must fully complete the equipment and is responsible for contacting the procurement manager to confirm all details, ensuring the delivered trimming lines are entirely functional and capable of fulfilling their assigned tasks. The offer must explicitly include: |
|  | • The complete line structure, including two working stations. |
|  | • All necessary main and waste conveyor belts, motors, and speed control systems. |
|  | • Optional features like individual trimming boards/inserts, subject to final design confirmation. |
|  | • Installation materials, utility connections, commissioning, and comprehensive staff training. |
| Documentation | To be supplied in Latvian or English: user manual, electric diagram, maintenance instructions, and CE Declaration of Conformity. |
| Guarantee | Minimum 24 months full manufacturer or supplier warranty. |