## Annex No. 2

## Technical Specification

### Purchase of Industrial Upright Refrigerated Cabinets

| Parameter | Requirement |
| --- | --- |
| THE PARTICIPANT shall provide information or proof against each requirement. |  |
| Quantity | 5 pcs. – Industrial Upright Refrigerated Cabinets (Single Door, 700L Class). |
| Purpose | Professional, high-capacity preservation and storage of food products (meat, dairy, prepared goods) at chilled temperatures, designed for heavy-duty use in warm environments. |
| 1. Dimensions and Capacity |  |
| * External Width (W) | 700 mm – 720 mm. |
| * External Depth (D) | 830 mm – 870 mm. |
| * External Height (H) | 2030 mm – 2080 mm. |
| * Nominal Net Volume | 470 L – 480 L. |
| 2. Temperature and Cooling |  |
| * Working Temperature Range | -2C to +8C. |
| * Climatic Class | Must be certified to Climatic Class 5 40C, 40% humidity. |
| * Refrigeration System | Patented and tropicalized monobloc refrigerating unit. |
| * Refrigerant Type | Hydrocarbon gas, specifically R290. |
| * Defrost/Condensation | Hot gas defrosting and condensation water evaporation. |
| 3. Construction and Hygiene |  |
| * Material | AISI 304 Stainless Steel monocoque structure. |
| * Insulation | High-density HFC-free polyurethane foam insulation (40 kg/cub.m). |
| * Interior Design | Chambers must have rounded internal edges for perfect cleaning and hygiene. |
| * Door Features | Self-closing doors (from 100 degrees); micro door sensor to stop the fan when the door is open; door fitted with a removable magnetic seal. |
| * Shelving | Must include GN 2/1 plasticized grids and removable racks (no tools needed). |
| * Base | Adjustable feet (h120/150 mm). |
| 4. Technical Requirements |  |
| * Supply Voltage | Standard power supply: 230V, 50\Hz single-phase. |
| * Total Power | Total power consumption max. 450 W to 800 W. |
| Complete Functional Set (Mandatory) | The Supplier must ensure the delivery of five fully functional cabinets. The offer must explicitly include: the complete cabinet units, self-closing doors, magnetic seals, GN 2/1 grids, and removable racks. |
| Documentation | To be supplied in Latvian or English: user manual, electric diagram, maintenance instructions, and CE Declaration of Conformity. |
| Guarantee | Minimum 24 months full manufacturer or supplier warranty. |