## Annex No. 2

## Technical Specification

### Purchase of Industrial Upright Refrigerated Fish Cabinet with Humidity Control

| Parameter | Requirement |
| --- | --- |
| THE PARTICIPANT shall provide information or proof against each requirement. |  |
| Quantity | 1 pc. – Industrial Upright Refrigerated Cabinet, specifically designed for fresh fish storage, processing, and preservation. |
| Purpose (Updated) | To store and preserve fresh fish by maintaining critical low temperatures and controlled humidity/air circulation. The unit must ensure optimal freshness, hygiene, and support short-term drying or preservation processes (e.g., curing/salting-prep stage) in accordance with HACCP principles. |
| 1. Dimensions and Capacity |  |
| * External Length (L) | 740 mm – 760 mm. |
| * External Depth (D) | 840 mm – 860 mm. |
| * External Height (H) | 2070 mm – 2100 mm. |
| * Door Configuration | Single hinged door. |
| 2. Temperature, Humidity, and Air Management (Updated) |  |
| * Working Temperature Range | -5C to +5C (low temperature range necessary for fresh fish). |
| * Humidity Range (Critical) | The cabinet must provide Active Humidity Control with an adjustable range of 40% - 90%. |
| * Ventilation | Forced air circulation with adjustable ventilation speed (HIGH/LOW) for controlled preservation and/or drying/aging (ventilation) management. |
| * Defrost System | Automatic defrosting. |
| 3. Construction and Hygiene (Critical) |  |
| * Material (Product Contact) | Interior and primary contact areas must be constructed from AISI 316 Stainless Steel or specialized, certified anti-corrosion AISI 304, capable of resisting salt, brine, and acidity exposure. |
| * Insulation | High-density foam insulation (minimum 70 mm thickness) for excellent thermal efficiency. |
| * Interior Design | The base of the cabinet must be structured as a watertight bottom/base tank designed to contain and drain melting ice or fish liquids. |
| * Internal Racks | Must include specialized, perforated draining baskets/trays or supports to allow liquids to separate from the product and collect in the base tank. |
| 4. Technical Requirements |  |
| * Supply Voltage | 400V / 3N PE / 50 Hz or equivalent industrial voltage. |
| * Controls | Digital electronic control panel for precise temperature and humidity monitoring and adjustment. |
| 5. Complete Functional Set (Mandatory) | The Supplier must fully complete the equipment, ensuring the delivered unit is explicitly ready for use as a high-hygiene fish storage and controlled preservation unit. The offer must explicitly include: |
|  | • The complete cabinet unit with integrated cooling system and active humidity management system. |
|  | • All necessary internal draining racks/baskets and bottom base liquid collection tank. |
|  | • Installation materials, final utility connections, commissioning, and comprehensive staff training. |
| Documentation | To be supplied in Latvian or English: user manual, electric diagram, maintenance instructions, and CE Declaration of Conformity. |
| Guarantee | Minimum 24 months full manufacturer or supplier warranty. |