## Annex No. 2

## Technical Specification

### Purchase of Integrated Metal Detector System (Conveyor Type)

| Parameter | Requirement |
| --- | --- |
| THE PARTICIPANT shall provide information or proof against each requirement. |  |
| Quantity | 1 pc. – Industrial Integrated Metal Detector System (Detection Head + Conveyor). |
| Purpose | Reliable detection and rejection of metallic contaminants (ferrous, non-ferrous, and stainless steel) in packaged and unpackaged food products moving along the production line, ensuring compliance with international food safety standards. |
| 1. Metal Detector Head Specifications |  |
| * Detection Technology | Must feature 3D detection and 4-quadrant technology for maximum sensitivity and stability against temperature or product changes. |
| * Product Range | High sensitivity across dry, moist, salty, or frozen goods. |
| * Programs/Storage | Storage capacity for up to 250 products. |
| * Controls | Easy setup and operation using auto-calibration, intuitive, and multilingual menus. |
| 2. Conveyor System (Conveyor HQ Class) |  |
| * Construction Material | 100% stainless steel construction for stability and hygiene. |
| * Integration | Conveyor must be optimally adapted for seamless use with the detection head. |
| * Belt Material | Must be supplied with a belt type (e.g., PU or chainlink) that is FDA compliant and heat resistant up to 120C. |
| * Hygiene Rating | Minimum IP65 protection class, with an option for Low Pressure Washdown (LPW) or High Pressure Washdown (HPW). |
| * Belt Speed | Adjustable belt speed range: 12 – 24 m/min. |
| * Height | Adjustable working height: 800 mm\pm 100 \tmm. |
| 3. Mandatory Accessories (Rejection & Control) |  |
| * Rejection System | Mandatory inclusion of an integrated automatic rejection system (e.g., AirJet, transversal/linear rejection unit, or flip arm) appropriate for the product size and type. |
| * Safety Accessories | Includes necessary railing systems and light barriers. |
| * Central Monitoring (Option) | Must be compatible with central monitoring software (SHARKNET or equivalent) for HACCP/IFS documentation (if required by procurement manager). |
| 4. Compliance and Safety |  |
| * Standards | Must comply with HACCP and IFS standards. CE marking is mandatory. |
| * Power / Voltage | 400 V, 3 Phase, 50 Hz (or compatible industrial voltage). |
| Complete Functional Set (Mandatory) | The Supplier must ensure that all components and preparation of the machine are completed for full operation. If data is lacking, the supplier must contact the procurement department. The offer must explicitly include: the detection head, the stainless steel conveyor, the automatic rejection unit, necessary control cabinet, and all internal cabling/software licensing for the stated functions. |
| Documentation | To be supplied in Latvian or English: user manual, electric diagram, maintenance instructions, and CE Declaration of Conformity. |
| Guarantee | Minimum 24 months full manufacturer or supplier warranty. |