## Annex No. 2

## Technical Specification

### Purchase of Custom Industrial Gentle Caviar Mixing System (Rotating Tub Type)

| Parameter | Requirement |
| --- | --- |
| THE PARTICIPANT shall provide information or proof against each requirement. |  |
| Quantity | 1 pc. – Custom-Designed Industrial Gentle Caviar Mixing System (Rotating Tub / Stirring Frame Concept or Equivalent). |
| Intended Use (Purpose - Critical) | The system is designed for gentle mixing of fish roe/caviar with salt, preservatives, and other ingredients, ensuring uniform distribution and preserving the roe structure (preventing crushing or shearing) over a short cycle time. The system must support production capacity of up to 4,000 kg/month. |
| 1. Technological Concept (MANDATORY) |  |
| * Mixing Principle | The system must be exclusively based on the "Rotating Tub / Stirring Frame" principle, where the frame mechanically rotates/tumbles the product container. |
| * Mixing Action | Mixing must occur through a gentle tumbling / rocking motion, with no internal mechanical mixing paddles or agitators contacting the product. |
| * Performance | Must ensure uniform ingredient distribution and prevent mechanical pressure or rupture of the roe grains. Typical cycle time: 1 minute – 3 minutes per batch. |
| 2. Stirring Frame (Main Unit) |  |
| * Construction Material | Robust frame made of AISI 304 Stainless Steel (AISI 316 preferred for surfaces subject to splashing/contact with salt). |
| * Rotation Control | Motorized rotation unit with adjustable, very low speed control (orientational range: 5 RPM – 25 RPM). |
| * Direction | Must provide reversible rotation capability (preferred). |
| * Controls | Control panel on the frame featuring ON/OFF, speed control, and emergency stop button. |
| * Hygiene | Unit must be wash-down ready (ATEX-rated electrical components or IP55/higher enclosure required) and suppress vibration/noise. |
| * Dimensions | Precise external dimensions (length, width, height, weight) to be provided by the bidder in their technical offering. |
| 3. Caviar Tubs (Mixing Containers - INCLUDED) |  |
| * Quantity Included | 1 pc. – 2 pcs. Stainless Steel Tubs (vessels). |
| * Material (Mandatory) | Tub material must be AISI 316 Stainless Steel (mandatory due to direct contact with brine/roe). |
| * Capacity | Nominal tub capacity: 40 L – 80 L (handling a roe batch of 20 kg – 50 kg). |
| * Design | Tub must be cylindrical or gently conical, with smooth internal bottom and no sharp edges. Must include external handles for transfer. |
| * Tub Fixing | Tub must be reliably fixed onto the frame using a quick mechanical clamp or locking system (compatible with the frame). |
| 4. Safety, Ergonomics, and Installation |  |
| * Safety | Must include protective guarding or a safety interlock system compliant with EU safety logic (EN 1672-2). |
| * Ergonomics | The unit height and fixing mechanism must allow the tub to be easily placed onto the frame by one operator. |
| * Mobility | Construction on wheels with brakes is preferred/optional. |
| Complete Functional Set (Mandatory - Critical) | The Supplier must ensure that all components and preparation of the machine are completed for full operation, including the supply of the main unit and the 1-2 specialized AISI 316 tubs. If data is lacking, the supplier must contact the procurement department to confirm necessary accessory systems (e.g., specific basket cranes, control modules, or safety sensors). |
| Documentation | Must include: Technical Data Sheet, Materials Specification, Operating Instructions, Safety Instructions, and CE Declaration of Conformity. |
| Guarantee | Minimum 24 months full manufacturer or supplier warranty. |