## Annex No. 2

## Technical Specification

### Purchase of Industrial Compact Depositing Machines (Tabletop Type)

| Parameter | Requirement |
| --- | --- |
| THE PARTICIPANT shall provide information or proof against each requirement. |  |
| Quantity | 5 pcs. – Industrial Compact Tabletop Depositing Machines. |
| Purpose | High-accuracy, flexible depositing of a wide variety of semi-liquid or viscous food products, including batter, mousses, cream, custard, fruit fillings, sauces, soups, and ready-made meals. Suitable for small to medium-sized operations (e.g., small bakeries, caterers, hotels). |
| 1. Core Specifications (Per Unit) |  |
| * Dosing Volume Range | 10 ml – 550 ml. |
| * Hopper Capacity | 24 Litres – 26 Litres. |
| * Depositing Cylinder | Must be equipped with a cylinder system for accurate volume control. |
| * Dosing Control | Dosing volume must be manually set and indicated on a scale. Dosing speed must be easily adjusted by a rotary knob. |
| * Assembly/Cleaning | Designed for fast and easy cleaning (less than a minute) without tools for dismantling. All cleanable parts must be dishwasher proof. |
| 2. Technical Requirements |  |
| * Operation | Pneumatically operated (air cylinder system). |
| * Operating Pressure | Requires operating pressure of 3.5 bar – 4.5 bar. |
| * Air Consumption | Air consumption at 60 strokes per minute must be no more than 65 L/min – 75 L/min. |
| * Construction Material | Built entirely of stainless steel and other non-rusting parts. |
| 3. Mandatory Accessory Sets (Included for EACH of the 5 units) |  |
| * Set A: Injection Needles | Must include the necessary equipment/model variant for applications requiring injection (e.g., jam, fruit fillings, custard, or cream into muffins, eclairs, or donuts). |
| * Set B: Hose and Handgun | Must include the necessary equipment/model variant with a hose and handgun for manual, versatile depositing (e.g., muffin batter, sauces, soups, ready-made meals). |
| * Set C: Dosing Curve | Must include the necessary equipment/model variant with a dosing curve or equivalent fixed dispensing mechanism for high-speed, continuous batch depositing. |
| * Base Control | Must include a foot control pedal (where applicable by design). |
| Complete Functional Set (Mandatory) | The Supplier must ensure the delivery of five fully functional depositing units, configured with all accessories necessary to perform the full range of applications (injection, handgun, and dosing curve). This implicitly includes a single base machine and all three required conversion kits (needles, hose/gun, and dosing curve) for each of the five machines. The offer must explicitly include: installation materials, pneumatic connections, commissioning, and comprehensive staff training. |
| Documentation | To be supplied in Latvian or English: user manual, maintenance instructions, and CE Declaration of Conformity. |
| Guarantee | Minimum 24 months full manufacturer or supplier warranty. |