## Annex No. 2

## Technical Specification

### Purchase of Industrial Complete Cleaning Stations with Hose Reel (Satellite System)

| Parameter | Requirement |
| --- | --- |
| THE PARTICIPANT shall provide information or proof against each requirement. |  |
| Quantity | 16 pcs. – Complete Cleaning Stations with Integrated Hose Reel (Satellite Stations). |
| Purpose | To function as a basic, non-mobile element of a central cleaning system. The station must provide all elements necessary for preliminary rinsing, foaming, disinfection, and final rinsing processes. |
| 1. System Type and Construction |  |
| * System Type | Designed as a satellite station (e.g., SR 25-1 class or equivalent) for connection to a central high-pressure water main. |
| * Material | Constructed from high-grade stainless steel (AISI 304 or equivalent) for exceptional durability, reliability, and chemical resistance. |
| * Design | Robust, hygienic design with convenience of use being paramount. |
| 2. Operational Performance |  |
| * Water Input Pressure | Designed to operate with a central water supply pressure of 20 bar – 30 bar. |
| * Water Flow Rate | Working flow rate: 12 L/min – 18 L/min (suitable for single-user intensive cleaning). |
| * Cleaning Stages | Must facilitate sequential stages: preliminary rinsing (with high-pressure water), foam application, intermediate rinsing, disinfectant application, and final rinsing. |
| 3. Mandatory Components (Complete Kit) |  |
| * Satellite Station | The core wall-mounted control/mixing unit (e.g., SR 25-1). |
| * Hose Reel | Integrated stainless steel hose reel (manual or automatic). |
| * Hose | Supplied with a minimum of 20 metres – 25 metres of high-pressure hose (double metal braided or equivalent industrial standard). |
| * Lances/Nozzles | A complete set of lances for three separate applications: washing/rinsing, foam application, and disinfection. |
| * Chemical Integration | Container hanger/basket for chemical agents (detergents/disinfectants). |
| * Accessories | Includes lance hanger, butterfly valves, quick couplings, and necessary connection hoses. |
| Complete Functional Set (Mandatory) | The Supplier must ensure that all components and preparation of the machine are completed for full operation. If data is lacking, the supplier must contact the procurement department. The offer must explicitly include: the satellite station, the hose reel, the high-pressure hose, and the complete set of lances/nozzles, fully installed and ready for connection to the central system. |
| Documentation | To be supplied in Latvian or English: user manual, electric diagram, maintenance instructions, and CE Declaration of Conformity. |
| Guarantee | Minimum 24 months full manufacturer or supplier warranty. |