## Annex No. 2

## Technical Specification

### Purchase of Industrial Electric Fry-Top Griddle on Open Stand

| Parameter | Requirement |
| --- | --- |
| THE PARTICIPANT shall provide information or proof against each requirement. |  |
| Quantity | 2 pcs. – Industrial Electric Fry-Top Griddles (Smooth Plate) on Open Stand. |
| Purpose | High-performance grilling and searing of meat, fish, and vegetables at precise temperatures (up to 300C), suitable for continuous use in a professional kitchen environment. Designed for seamless integration into a modular cooking line. |
| 1. Griddle Unit Specifications (Fry-Top) |  |
| * External Width (W) | 690 mm – 810 mm. |
| * External Depth (D) | 720 mm – 740 mm (Standard 700 Series depth). |
| * External Height (H) (Top Unit Only) | 280 mm – 300 mm. |
| * Plate Type | Smooth (L) mild steel or chrome-plated surface plate. |
| * Plate Thickness | Griddle plate thickness: 10 mm – 15 mm (to ensure rapid heating and recovery). |
| * Temperature Range | Thermostatic temperature control between 100C and 300C. |
| * Heating Zones | Single heating zone. |
| * Drainage | Includes a tilted surface toward the front, an opening to collect cooking grease, and a removable collection tray. |
| * Compliance | IPX5 grade water protection. |
| 2. Supporting Stand Specifications (Open Base) |  |
| * Compatibility | Stand must be designed to perfectly accommodate a 400 mm wide top module. |
| * Stand Material | Manufactured with a solid structure in AISI-304 stainless steel. |
| * Stand Dimensions | External Width (W): 390 mm – 410 mm. External Depth (D): 720 mm – 740 mm. External Height (H): 550 mm – 570 mm. |
| * Stand Functionality | Serves as an open storage element. Total height of assembled unit (Fry-Top + Stand) must be approx. 850 mm – 900 mm. |
| * Feet | Must include adjustable feet for precise height adjustment. |
| 3. Technical and Electrical |  |
| * Power / Load (Top Unit) | Electric connected load: 4.5 kW – 5.5 kW. |
| * Power Supply | 400V, 3 Phase + Neutral, 50 Hz. |
| Complete Functional Set (Mandatory) | The Supplier must ensure the delivery of two fully assembled, ready-to-use units, consisting of the Fry-Top unit and the Open Stand. The offer must explicitly include: the Fry-Top unit, the integrated grease collection tray, the open stand, and all necessary connection materials. |
| Documentation | Turi būti pateikta latvių arba anglų kalba: vartotojo vadovas, elektrinė schema, techninės priežiūros instrukcijos ir CE Atitikties Deklaracija. |
| Guarantee | Mažiausiai 24 mėnesių pilna gamintojo arba tiekėjo garantija. |