## Annex No. 2

## Technical Specification

### Purchase of Industrial High-Accuracy Mobile Depositing Machine

| Parameter | Requirement |
| --- | --- |
| THE PARTICIPANT shall provide information or proof against each requirement. |  |
| Quantity | 1 pc. – Industrial High-Accuracy Mobile Depositing Machine. |
| Purpose | Designed for precise, high-performance depositing of homogeneous and heterogeneous mixtures (e.g., cake batter, muffin batter, sponge cake, mousses, sauces, custards, and all kinds of fruit fillings) directly from a supply container or bowl. |
| 1. Operational Concept |  |
| * Dosing Principle | High-performance depositing providing great accuracy. |
| * Product Source | Capable of depositing directly from any bowl, container, or pail placed underneath the machine. |
| * Delivery System | Equipped with a flexible hose and handgun for controlled manual depositing of individual products (e.g., muffins, cupcakes, desserts). |
| * Dosing Control | Dosing volume must be manually set (indicated on a scale). Dosing speed must be easily adjustable via a rotary knob. |
| * Dosing Range (Nominal) | Designed for dosing volumes in the approximate range of 5 ml – 1000 ml (per cycle). |
| 2. Construction and Mobility |  |
| * Material | Built of stainless steel and other non-rusting materials. |
| * Mounting | Supplied as a stand-alone unit on a robust, hygienic mobile frame. |
| * Dimensions (Estimated) | Length: 700 mm – 900 mm. Width: 600 mm – 750 mm. Height: 1400 mm – 1600 mm. |
| * Hygiene | All construction materials must be suitable for the food industry and easily cleanable. |
| 3. Technical Requirements |  |
| * Height Setting | Must include a pneumatic height setting system (e.g., foot lever activated) to easily lower the suction tube into the product container. |
| * Pneumatic Supply | Requires compressed air operating pressure of 4 bar – 6 bar. |
| * Electrical Power | Max. connected load: 0.5 kW – 1.0 kW. |
| * Electrical Supply | Single-phase 230V, 50 Hz (or industrial three-phase equivalent, to be confirmed). |
| 4. Complete Functional Set (Mandatory) | The Supplier must ensure the delivery of a single fully functional depositing unit, including: |
|  | • The complete depositing machine (main unit) with hose and handgun. |
|  | • The pneumatic height adjustment system. |
|  | • All necessary components for high-accuracy dosing. |
|  | • Installation materials, final utility connections, commissioning, and comprehensive staff training. |
| Documentation | To be supplied in Latvian or English: user manual, electric diagram, maintenance instructions, and CE Declaration of Conformity. |
| Guarantee | Minimum 24 months full manufacturer or supplier warranty. |