## Annex No. 2

## Technical Specification

### Purchase of Industrial Insulated Cooling Vat for Caviar/Food Processing

| Parameter | Requirement |
| --- | --- |
| THE PARTICIPANT shall provide information or proof against each requirement. |  |
| Quantity | 1 pc. – Industrial Vertical Cooling Vat with Integrated Refrigeration Unit. |
| Purpose | Designed for rapid, stable cooling and controlled storage/stirring of delicate food products, such as caviar, sauces, or creams, requiring homogeneous temperature distribution and strict cold chain maintenance. |
| 1. Capacity and Dimensions |  |
| * Nominal Volume | 50 Litres – 55 Litres |
| * External Width (W) | 500 mm – 540 mm. |
| * External Depth (D) | 550 mm – 580 mm. |
| * External Height (H - Closed) | 1030 mm – 1070 mm. |
| 2. Cooling and Temperature Control |  |
| * Refrigeration Unit | Mandatory inclusion of an integrated refrigerating unit. |
| * Target Temperature | Capable of achieving and maintaining a stable working temperature in the range of 0C to +4C. |
| * Insulation | Double-walled insulated construction. |
| * Temperature Monitoring | Must include a functional measuring rod (measuring stick) and digital display/sensor for temperature control. |
| * Agitator/Stirrer | Includes an integrated agitator motor/stirrer for homogeneous cooling. |
| * Agitator Safety | The stirring mechanism must include a safety feature (e.g., gravity switch) that automatically stops the agitation if the lid is opened. |
| 3. Construction and Hygiene |  |
| * Material (Minimum) | Manufactured from high-quality Stainless Steel (AISI 304 minimum). Inner tank must be highly corrosion-resistant for use with food/brine solutions. |
| * Lid | Includes a hinged stainless steel lid with a min 20 cm diameter opening. |
| * Drainage Outlet | Must include a hygienic milk/product outlet connection (e.g., DN50, SMS51, or Triclamp outlet with stainless steel throttling valve). |
| * Base | Equipped with adjustable support feet for leveling the vat on uneven floors. |
| 4. Electrical Requirements |  |
| * Supply Voltage | 230 V, 50 Hz.. |
| * Electrical Safety | Electrical circuits must be protected against water or steam ingress. |
| Complete Functional Set (Mandatory) | The Supplier must ensure the delivery of a single fully functional unit, complete with the integrated refrigeration unit, agitator, measuring stick, and adjustable feet. The offer must explicitly include: installation materials, commissioning, and comprehensive staff training. |
| Documentation | To be supplied in Latvian or English: user manual, electric diagram, maintenance instructions, and CE Declaration of Conformity. |
| Guarantee | Minimum 24 months full manufacturer or supplier warranty. |