## Annex No. 2

## Technical Specification

### Purchase of Industrial One-Sided Cutting Tables with Undershelf

| Parameter | Requirement |
| --- | --- |
| THE PARTICIPANT shall provide information or proof against each requirement. |  |
| Quantity | 5 pcs. – Industrial One-Sided Cutting Tables on Legs with Undershelf (Welded or Screwed version). |
| Purpose | Single-operator workstation optimized for repetitive cutting, boning, and trimming tasks in food processing, providing a dedicated, hygienic, and replaceable plastic cutting surface. |
| 1. External Dimensions |  |
| * External Length (L) | 1980 mm – 2020 mm. |
| * External Depth (D) | 690 mm – 710 mm. |
| * External Height (H) | 880 mm – 920 mm. |
| 2. Construction and Worktop |  |
| * Material | Manufactured entirely from AISI 304 Stainless Steel. |
| * Surface Finish | Worktop surface must have a grained finish. |
| * Cutting Board Integration (Critical) | The worktop must be fitted with a dedicated channel/groove designed to hold a removable PE plastic cutting board. |
| * Cutting Board Dimensions | The integrated system must accommodate a board with a nominal width of 290 mm – 310 mm and a height/thickness of 28 mm – 32 mm. |
| * Raised Edge | Includes a rear upstand (raised edge) with a height of 45 mm – 55 mm. |
| * Undershelf (Mandatory) | Must include an adjustable stainless steel undershelf for storage. |
| * Assembly | Available in fully welded or screwed/disassembled version. |
| * Base | Must include adjustable feet (e.g., plastic adjustable glides) for precise height and level adjustment. |
| 3. Compliance |  |
| * Standards | Must comply with applicable EU food contact material regulations and hygiene standards (HACCP). |
| Complete Functional Set (Mandatory) | The Supplier must ensure the delivery of five complete and fully functional table units, including the worktop, legs, adjustable feet, and the mandatory bottom shelf. The price must include a single PE plastic cutting board for each table (5 units). |
| Documentation | To be supplied in Latvian or English: assembly instructions, and CE Declaration of Conformity. |
| Guarantee | Minimum 24 months full manufacturer or supplier warranty. |