## Annex No. 2

## Technical Specification

### Purchase of Industrial Pastry Work Tables with Solid Wood Top

| Parameter | Requirement |
| --- | --- |
| THE PARTICIPANT shall provide information or proof against each requirement. |  |
| Quantity | 2 pcs. – Industrial Pastry Work Tables on Stainless Steel Frame with Solid Hardwood Worktop. |
| Intended Use (Purpose) | Professional handling of pastry and laminated-dough, shaping, and general confectionery work, ensuring a hygienic and stable surface in a HACCP-compliant production environment. |
| 1. External Dimensions (Adjustable) |  |
| * External Length (L) | 1990 mm – 2010 mm. |
| * External Width (W) | 790 mm – 810 mm . |
| * External Height (H) | 840 mm – 880 mm (Height adjustable via feet). |
| * Load Capacity | Minimum static load capacity: 150 kg. Worktop deflection under full load: < 3 mm. |
| 2. Worktop Specifications (Critical) |  |
| * Worktop Material (Mandatory) | Solid Beech or Maple Hardwood Block (Food-grade certified). |
| * Worktop Thickness (Mandatory) | Minimum thickness of 50 mm. |
| * Surface Finish | Smooth, joint-free surface (max gap < 2 mm). Surface must be sealed with food-safe mineral oil or polyurethane suitable for direct food contact. |
| * Moisture Resistance | Underside of the wooden top must be sealed to prevent moisture absorption. |
| 3. Frame and Structure |  |
| * Frame Material | AISI 304 Stainless Steel (legs, frame, support bars). |
| * Welding Quality | Welding must be continuous TIG welded, with no crevices or dead zones. |
| * Base / Legs | Hygienic, open leg profile design, allowing full floor cleaning clearance. Equipped with food-industry adjustable feet (non-slip, stainless steel) providing min. 30 mm height adjustment. |
| 4. Hygiene, Sanitation, and Compliance |  |
| * Hygienic Design | Full compliance with HACCP, ISO 22000, and EHEDG hygienic design principles. Surfaces must be fully accessible for cleaning from all sides. |
| * Cleaning Tolerance | Suitable for repeated cleaning with mild alkaline detergents and warm water (up to 60C). |
| * Compliance Standards | Must comply with EU Regulation EC 1935/2004 and EN 1672-2. |
| * Marking | CE marking is mandatory. |
| Complete Functional Set (Mandatory) | The Supplier must ensure the delivery of two fully functional pastry tables, including the solid hardwood worktop and the full stainless steel frame/legs. The offer must explicitly include: |
|  | • The complete table structure with adjustable feet. |
|  | • Declaration of conformity for the wood treatment oils/varnish (food safety). |
|  | • Material certificates for AISI 304 stainless steel. |
| Documentation | To be supplied in Latvian or English: user manual, assembly instructions, and CE Declaration of Conformity. |
| Guarantee | Minimum 24 months full manufacturer or supplier warranty. |