## Annex No. 2

## Technical Specification

### Purchase of Industrial Waste Disposal Coolers

| Parameter | Requirement |
| --- | --- |
| THE PARTICIPANT shall provide information or proof against each requirement. |  |
| Quantity | 4 pcs. – Industrial Waste Disposal Coolers (Waste Bin Cooling Cabinets). |
| Purpose | To hygienically cool and store food waste, preventing bacterial growth and minimizing odors by maintaining controlled low temperatures. Designed for accepting and cooling two standard large-volume waste bins. |
| 1. Capacity and Dimensions |  |
| * Waste Bin Capacity | Designed to accommodate two (2) waste bins of 240 Litres each. |
| * Total Gross Capacity | 1100 Litres – 1150 Litres. |
| * External Length (L) | 1700 mm – 1720 mm. |
| * External Depth (D) | 860 mm – 880 mm. |
| * External Height (H) | 1190 mm – 1210 mm. |
| 2. Temperature and Cooling System (Self-Contained) |  |
| * Working Temperature Range | +2C to +15C. |
| * Ambient Operation | Suitable for ambient temperatures ranging from +15C to +40C. |
| * Refrigeration Unit | Integrated, self-contained refrigerating unit (Built-in cooling). |
| * Refrigerant Type | R290 (Hydrocarbon, low GWP). |
| * Defrost System | Automatic defrosting and evaporation of defrost water using hot gas. |
| 3. Construction and Technical Requirements |  |
| * Material | Constructed from Stainless Steel. |
| * Evaporator | Evaporator system must be foamed-in (e.g., in the stainless steel back wall). |
| * Access | Equipped with a hinged poor-in aperture (lid) for waste disposal access. |
| * Controls | Must feature electronic control. |
| * Power / Load | Connected load: 350 W – 450 W. |
| * Electrical Supply | 230 V, 50 Hz. |
| Complete Functional Set (Mandatory) | The Supplier must ensure the delivery of four fully functional Waste Disposal Cooler units, complete, assembled, and ready to plug in. The offer must explicitly include: the cooler unit, integrated refrigerating unit, all internal components, and the automatic defrost/evaporation system. |
| Documentation | To be supplied in Latvian or English: user manual, electric diagram, maintenance instructions, and CE Declaration of Conformity. |
| Guarantee | Minimum 24 months full manufacturer or supplier warranty. |