| CLARIFICATION OF THE TECHNICAL SPECIFICATION Procurement ID No. 24022026-01 |  |  |  |
| --- | --- | --- | --- |
| Lot No. 9 |  |  |  |
| 9.1. | High-Performance Warehouse Reach Truck | 1 | - Battery type – Lithium-ion, not less than 300 Ah - Battery installation – Rail-mounted - Mast – 3-stage mast, lifting height 10000 mm or higher, with free lift; mast height in lowered position not exceeding h1 = 3950 mm - Maximum load capacity at maximum lifting height – not less than 900 kg - Distance between support forks – 900 ± 40 mm - Hydraulic control - Steering angle – 360° - Overhead operator protective guard - Fork length – 1150 ± 10 mm - With side shift - Fork tilt – 4° forward, 2° backward - User manual in Latvian language and electronic format - Load backrest – 1000 mm - Charger included - Automatic 0-position fork tilt - Automatic 0-position side shift - Blue LED safety light (in the direction of the drive wheel) - Height indicator - Fork positioning laser - Camera system - Armrest - DC/DC converter 48V/12V, with outlet on the control panel - Delivery, installation and staff training included. |