PROCUREMENT TECHNICAL SPECIFICATION – REGULATIONS

“Supply of integrated industrial air filtration, safety, and air humidification systems with the implementation of digitalization and artificial intelligence (AI) solutions

for SIA “Dižozols Plus” needs”

# Procurement identification No. 15042026-01

## Acone

## 2026

GENERAL INFORMATION ABOUT PROCUREMENT PROCEDURE

Procurement “Supply of integrated industrial air filtration, safety, and air humidification systems with the implementation of digitalization and artificial intelligence (AI) solutions for SIA “Dižozols Plus” needs”, Procurement identification No 15042026-01, is organised within the project “Support for digitalisation of business processes in Latvia” No. 1.2.2.1/1/25/I/001, implemented with the framework of the specific support objective 1.2.2. “Use the advantages of digitalization for business development” of the European Union Cohesion Policy Programme 2021–2027, measure 1.2.2.1. “Support for the digitalization of processes in commercial activities”.

The procurement procedure is organised in accordance with the Cabinet of Ministers Regulation No. 104 of 28 February 2017 “Regulations on procurement procedure and its application to projects financed by the client”, applicable to projects financed from EU funds and Recovery and Resilience Facility support programmes.

- Support applicant: Dižozols plus LLC

- VAT number: LV400033784682

- Address: Granīta iela 32 k-13, Acone, Salaspils pagasts, Salaspils novads, LV-2119

- Procurement subject: integrated industrial air filtration, safety and air humidification systems with digitalization and artificial intelligence solutions delivery, installation and commissioning for woodworking production processes.

- Delivery address: Granīta iela 32 k-13, Acone, Salaspils pagasts, Salaspils novads, LV-2119, Latvia

- Delivery time: 30th of April 2027

- Offer validity due date: At least 180 days from offer submission. When submitting an offer, the applicant hereby confirms that the submitted offer is valid for 180 days from the date of submission of the offer. If the Customer cannot sign procurement contract during the validity period of the offer due to objective reasons, Customer can request applicant in writing form to extend their offer validity period.

| Criteria for the Applicant: | - The applicant must have experience with at least 2-3 similar projects in the supply, installation or maintenance of industrial equipment, dust extraction systems, air filtration, air humidification or industrial automation systems. - The applicant's average annual turnover over the last 3 calendar years is not less than the total value of the offer. - The applicant must ensure the availability of technical support and maintenance services in the territory of Latvia, ensuring a service response time of no more than 24 hours on working days. - The proposed solution must provide a fully integrated industrial system that includes wood dust collection and filtration; safety systems (including spark extinguishing); air humidification system, ensuring continuous operation in the woodworking environment. - All equipment must be mutually compatible and operate as a single system. - The offered equipment cannot be previously used, they cannot contain built-in used or refurbished components. - The Applicant must ensure compliance with applicable European Union safety, industrial and environmental standards. |  |
| --- | --- | --- |
| Procurement subject description: |  |  |
| General conditions | - All equipment should provide maximum throughput rate. - Insurance, transport, installation and training cost should be included in the price offer. - Equipment manuals should be on English or Latvian. - The offered equipment must be compatible with industrial wood processing environments and designed for continuous operation. - All costs related to transport, installation, integration, testing and commissioning must be included in the price offer. - The values ​​of the technical parameters must not be lower than those specified in the technical specification. Deviations of parameters up to ±5% are permissible, except for critical parameters (volume, safety systems, ATEX elements), for which deviations are not permissible. |  |
| Compliance with European Union standards in operation and legislation of the Republic of Latvia | - Certificate of compliance; - Applicable European Union directives and safety standards; - CE conformity requirements for industrial equipment; - Applicable ATEX requirements where relevant for dust explosion risk environments; - Environmental and occupational safety regulations applicable in the Republic of Latvia. |  |
| Tender Compliance | If there is a possible equivalent to the requirements set out in the procurement subject description, the existence of which has not been foreseen by the Customer, the Applicant may submit a tender compliant with the equivalent requirements. The Applicant may also submit a tender compliant with higher requirements. |  |
| Equipment supply, installation and launching | Must be provided. |  |
| Staff training | Must be provided. |  |
| Service | Must be able to provide a service response time of 24 hours in the Republic of Latvia and support during the warranty period. |
| Service availability | Equipment parts that are subjected to natural depreciation must be accessible in the supplier’s warehouse, ensuring their delivery to the customer within one working day. |
| Warranty | Not less than 12 (twelve) months from the date of signing the Acceptance and Handover Act. |
| Work performed by the supplier: | Delivery, installation, commissioning and service provision during operation. |
| Additional rules | Contest participants have the right to submit an offer for each individual full lot, each offer will be evaluated at the lowest price for each full lot separately. |

Procurement subject description:

| No. | Equipment name | Quantity Unit / set | Technical specification |
| --- | --- | --- | --- |
| Lot No. 1 Industrial dust collection, storage, safety and AI management system |  |  |  |
| 1. | Storage Silo | 1 | Specialized container designed to safely collect, store, and discharge large volumes of wood dust and sawdust generated from woodworking processes. Requirements: - Cylindrical for stress distribution, better resistance and fewer weak points. - Hot dip galvanized S235 JR carbon steel. - Not more than Ø 5,000 mm diameter. - Not more than 11,000 mm height. - Capacity not less than 140 m³. - Flooding water ring built in galvanized steel, to be connected to the water supply of the company, equipped with nozzles for silo extinguishing. Ladder and hoop terrace. |
| 2. | Storage Silo Operational Accessories and Sensors | Set | Auxiliary equipment ensuring the safe and continuous operation of a silo. It comprises monitoring devices, safety elements and material discharge mechanisms that operate without interrupting main plant processes. Required components: - Filling level sensor which monitors the volume of material inside the silo to prevent overfilling. - A minimum of 3 separate outlets allowing simultaneous and autonomous routing of wood chips. - Rotary valve suitable to exhaustion with running plant, complete with its own motor gearbox, allowing simultaneous and autonomous routing of wood chips. - “Venting” system (explosion relief panels), designed to safely release pressure during a dust explosion, sized strictly according to ATEX safety regulations. - Vibrations sensors installed on the extraction fans to detect abnormal shaking and prevent mechanical failures. - Temperature sensor which monitors the internal temperature of the system to provide early warning of potential fire risks. 1 sloped auger for dust discharge into metal bin, with safety limit switch. |
| 3. | Cyclones | 2 | Pre-filtration devices which use centrifugal force to separate large particles (like wood chips and sawdust) from the air stream. Requirements: - Must be installed on silo top. - 800 x 1000 mm diameter. - Hot dip galvanized. EV40 roto-valves installed at cyclones outlets. |
| 4. | Control of the electric equipment | 1 | A centralized control system designed to process sensor inputs and automatically manage the operation of all system components. It must include the following: - Colour touch screen not less than 10’. - Breaker protection of all supplied equipment. - Compliant to the INDUSTRY 4.0 which is used to monitor and control the plant running parameters. It includes: - Possibility of connecting to the system from the local company network, via Ethernet bus - Possibility of connecting to the system in mobile way, via smartphone or pc/tablet. - Display of the following parameters. - System vacuum. - Value of dust in the atmosphere, with relative limits and alarms. - Electric energy consumption value for each single electro-fan and total system. - Worked hours. - Alarm reports. - Possibility to export the data listed above for report on file in .csv format on USB support. - Possibility of remote connection for software updates / system monitoring. |
| 5. | AI-based functionalities of monitoring system | 1 | A material storage electrical control system with an AI agent. Integration of the following subsystems into a unified intelligent platform: - pneumatic transport system; - spark detection and monitoring system; - chip/sawdust storage system; This integrated approach ensure coordinated system operation, reduced energy consumption, and improved remote monitoring capabilities. Additionally, the AI agent provide the following capabilities: - Use of machine learning algorithms to: - analyze spark occurrence locations and root causes - predict cutting tool wear as a potential source of sparks - Perform data analysis and model development to support: - predictive maintenance - timely tool replacement - Continuous monitoring and analysis of: - Vibration - Temperature - other operational parameters - deviations from normal operating conditions This enable: - early detection of potential failures - extended equipment lifetime - reduced fire risks - minimization of unexpected downtime |
| 6. | Spark Detection and Extinguishing System | Set | Requirements: - Minimum of 3 control units to process detector signals and automatically trigger the extinguishing mechanisms. - Spark detectors: ATEX II 3D rated, IP65 optical sensors with a 90° viewing angle and a reaction time of ≤ 40 ms for instantaneous spark detection. - Water spray nozzles: ATEX II 3D rated extinguishing nozzles, equipped with 24 VDC electro-valves and integrated water pressure probes for immediate fire suppression. - Minimum of 3 alarm sirens devices to alert plant personnel of safety system activation. Minimum of 4 standalone pressurization units, each featuring a ≥ 40-liter stainless-steel tank, to ensure constant, reliable water pressure for the anti-spark system. |
| 7. | Heating pipe For outdoor use only | 4 | 4 sets of trace heating systems, each 18 meters in length, dedicated to the 4 outdoor extraction lines. Designed to prevent internal condensation, freezing, and material clogging during cold weather. |
| 8. | System installation and operational training for personnel. |  |  |

| No. | Equipment name | Quantity Unit / set | Technical specification |
| --- | --- | --- | --- |

| Lot No. 2 Air humidification system |  |  |  |
| --- | --- | --- | --- |
| 1. | Air Humidification System | set | The air humidification system is designed to maintain the required relative humidity level in industrial woodworking premises, ensuring stable microclimate conditions and reducing dust dispersion. The system shall provide automatic humidity control according to specified parameters, taking into account ventilation intensity, air exchange rate, and external climatic conditions. The Tenderer may offer a technologically equivalent or superior solution that ensures the required performance and functionality. Room No. 1: Air extraction – 25,000 m³/h Recirculation – 0 m³/h Air volume to be humidified – 62,500 m³ Indoor air temperature – 23°C / 30°C Minimum/average outdoor temperature – -15°C (80% RH) Natural ventilation – up to 20%/h Required relative humidity – 45% RH Calculated system capacity – 80.7 l/h Room No. 2: Air extraction – 0 m³/h Recirculation – 0 m³/h Air volume to be humidified – 3,224 m³ Indoor air temperature – 23°C / 30°C Minimum/average outdoor temperature – -15°C (80% RH) Natural ventilation – up to 40%/h Required relative humidity – 45% RH Calculated system capacity – 38.6 l/h Room No. 3: Air extraction – 0 m³/h Recirculation – 0 m³/h Air volume to be humidified – 2,728 m³ Indoor air temperature – 23°C / 30°C Minimum/average outdoor temperature – -15°C (80% RH) Natural ventilation – up to 40%/h Required relative humidity – 45% RH Calculated system capacity – 25.9 l/h Room No. 4: Air extraction – 30,000 m³/h Recirculation – 0 m³/h Air volume to be humidified – 7,511 m³ Indoor air temperature – 23°C / 30°C Minimum/average outdoor temperature – -15°C (80% RH) Natural ventilation – up to 20%/h Required relative humidity – 45% RH Calculated system capacity – 72.6 l/h Room No. 5: Air extraction – 100,000 m³/h Recirculation – 0 m³/h Air volume to be humidified – 40,000 m³ Indoor air temperature – 23°C / 30°C Minimum/average outdoor temperature – -15°C (80% RH) Natural ventilation – up to 20%/h Required relative humidity – 45% RH Calculated system capacity – 388.1 l/h Room No. 6: Air extraction – 60,000 m³/h Recirculation – 0 m³/h Air volume to be humidified – 15,000 m³ Indoor air temperature – 23°C / 30°C Minimum/average outdoor temperature – -15°C (80% RH) Natural ventilation – up to 20%/h Required relative humidity – 45% RH Calculated system capacity – 145.6 l/h Room No. 7: Air extraction – 20,000 m³/h Recirculation – 0 m³/h Air volume to be humidified – 20,000 m³ Indoor air temperature – 23°C / 30°C Minimum/average outdoor temperature – -15°C (80% RH) Natural ventilation – up to 20%/h Required relative humidity – 45% RH Calculated system capacity – 194.1 l/h Room No. 8: Air extraction – 0 m³/h Recirculation – 0 m³/h Air volume to be humidified – 1,568 m³ Indoor air temperature – 23°C / 30°C Minimum/average outdoor temperature – -15°C (80% RH) Natural ventilation – up to 40%/h Required relative humidity – 45% RH Calculated system capacity – 35.6 l/h The scope shall include delivery, installation, commissioning, and personnel training. |

General conditions:

- When submitting the final offer for all equipment, the manufacturer/brand/model and price must be specified;

- All spare parts /additional elements must be included;

- If the offer includes additional equipment, it should be listed;

- The offered equipment must be suitable for operation in industrial wood processing environments, including handling of wood dust, chips and technological waste;

- The proposed solution shall function as a fully integrated and interconnected system, including air filtration equipment, spark detection and extinguishing system, electrical control system, as well as digitalization and artificial intelligence (AI) components, ensuring centralized data processing, analysis, and remote monitoring;

- All costs related to transportation, delivery, installation, system integration, testing and commissioning must be included in the offered price.

- All references to specific standards, goods or manufacturers used in the procurement documentation may be replaced by the equipment supplier with equivalent or analogous products or goods that meet the requirements of the specification.

- The values ​​of the technical parameters must not be lower than those specified in the technical specification. Deviations of the parameters up to ±5% are permissible, except for critical parameters (safety systems, volume, ATEX elements), for which deviations are not permissible.

- Warranty: Not less than 12 (twelve) months from the date of signing the Acceptance and Handover Act.

- By submitting the offer, the applicant has confirmed that the warranty period of the offered goods is at least 12 months.

- Requirements for preparation and submission of the final offer:

- Information about the supplier (company details);

- Expected term of contract execution – 30.04.2027.;

- Offer validity period – 180 days;

- Offer price in EUR without VAT;

- Date of offer, name, surname, position, signature, contact information;

- The offer is prepared electronically, including all required information.

- Applicants must fill in and submit Appendix 2 of the procurement - Certificate of an independently developed offer.

- Suppliers may submit their questions no later than 30th of April 2026 at 17:00. Questions received after this time will not be accepted or considered.

  - Suppliers shall submit their final offers by sending them electronically to the following email addresses: girts@dizozols.lv  and ingus@1b.lv no later than 17:00 on 07th of May 2026, ensuring confirmation of receipt, either by a message from the recipient or by an email delivery receipt from the email service provider. The offer must be certified with a secure electronic signature and a timestamp.

  - The offer must be prepared and submitted in Latvian or English.

  - The validity period of the offer must be stated in the offer and shall be no less than 180 days from the deadline for submission of offers.

- Phone number for more information: Ingus Diķis; ingus@1b.lv, +371-26468611.

Dižozols plus LLC

Member of the Board

Ģirts Plakans

* THIS DOCUMENT IS SIGNED WITH A SECURE ELECTRONIC SIGNATURE AND CONTAINS A TIMESTAMP.

| ##### Pielikums Nr. 1. Līguma paraugs ##### PIRKUMA - PĀRDEVUMA L Ī G U M S Nr. _/20___ Šis līgums noslēgts Rīgā, 202_.gada xx.xxx starp xxxxxxxxx reģ. Nr. xxxxxxxxxxxxxxxxxxx, turpmāk tekstā Pārdevējs, tās valdes priekšsēdētāja xxxx xxxxx personā, kas darbojas uz statūtu pamata un SIA „xxxxxxxxxxx” reģ. Nr. xxxxxxxxxxxxxxxx, turpmāk tekstā Pircējs, tās valdes priekšsēdētājas xxxxxxxxxxxxxxxxx personā, kas darbojas uz statūtu pamata, ŅEMOT VĒRĀ, KA: 1.Pircējs apliecina, ka iekārtas iegāde tiek veikta Eiropas Savienības kohēzijas politikas programmas 2021.–2027. gadam 1.2.2. specifiskā atbalsta mērķa “Izmantot digitalizācijas priekšrocības uzņēmējdarbības attīstībai” 1.2.2.1. pasākuma “Atbalsts procesu digitalizācijai komercdarbībā” ietvaros. Pircēja projekta iesniegums Nr. xxxxxxxxxxxxxxxxx. 2. Pārdevējs ir atzīts par uzvarētāju iepirkuma procedūrā,…………………………... 1. Līguma priekšmets. 1.1. Pārdevējs pārdod, piegādā un uzstāda, bet Pircējs saskaņā ar šī līguma noteikumiem pērk iekārtu komplektu, kuras komplektācija, tehniskie rādītāji, opcijas, cena, izgatavošanas, piegādes termiņi un apmaksas noteikumi tiek noteikti Pircējam piedāvājumā, kurš ir Līguma pielikums Nr. 1 un tā neatņemama sastāvdaļa, turpmāk tekstā Iekārta. 1.2. katras atsevišķas iekārtas piegāde un uzstādīšana notiek atbilstoši projekta plānam un pielikumam Nr.2, kas ir līguma neatņemama sastāvdaļa. 2. Cenas un līguma kopējā vērtība. Līguma kopējā summa (pirkuma maksa) ir EUR XXXX (XXXXXXXXXXeiro, 00 centi). Pirkuma maksā ir iekļauta iekārtas iepakošana, piegāde, uzstādīšana un darbinieku vienreizēja apmācība. 3. Apmaksas un piegādes noteikumi. 3.1.Pircējs apņemas veikt priekšapmaksu par katru Iekārtu xx % apmērā no kopējās Līguma. Iekārtas izgatavošana tiek uzsākta no brīža, kad nauda ir ieskaitīta Pārdevēja kontā. 3.2.Nākamo apmaksu xx% apmērā, pircējs apņemas veikt nekavējoties pēc paziņojuma par iekārtas gatavību un rēķina saņemšanas. Iekārta no izgatavotājrūpnīcas tiek izsūtīta tikai pēc šā maksājuma saņemšanas. 3.3. Pēdējo apmaksu xx% pircējs apņemas veikt 5 (piecu) darba dienu laikā pēc iekārtas uzstādīšanas un pieņemšanas. Iekārtu pieņemot tiek parakstīts pieņemšanas-nodošanas akts. 3.4.Par apmaksas dienu tiek uzskatīta diena, kad nauda ir ieskaitīta Pārdevēja kontā. 3.5. Pārdevējs apņemas nodrošināt Iekārtu izgatavošanu un piegādi atbilstoši pielikumam Nr.2 pie šī līguma 3.1. un 3.2 punktu noteikto darbību izpildes un pie nosacījuma, ka 3.2. punktā paredzēta apmaksa tiek veikta bez kavēšanās. 3.7. Pārdevējs nodrošina iekārtas piegādi līdz Pircēja teritorijai, uzstādīšanu un darbinieku vienreizēju apmācību. Pircējs nodrošina iekārtas pārvietošanu teritorijā un novietošanu ražošanas telpās (klātesot pārdevēja pārstāvim), kā arī nepieciešamos elektrības, tvaika ,kanalizācijas, gāzes un saspiestā gaisa pievadus līdz iekārtai. 3.8.Iekārtu piegādes un uzstādīšanas vieta ir xxxxxxx. 3.9. Pēc Iekārtas piegādes, Pārdevējs veic iekārtas uzstādīšanas atbilstoši pielikumam Nr.2. 3.10. Īpašumtiesības uz Iekārtu pāriet no Pārdevēja uz Pircēju PNA parakstīšanas brīdī. Preces atbilstību PNA norādītajam Pircēja pilnvarotais pārstāvis apstiprina ar savu parakstu uz PNA. 3.10.1.Pēc pieņemšanas – nodošanas akta parakstīšanas Pircējam vairs nav tiesību izteikt pretenzijas attiecībā uz Iekārtas komplektāciju. - Pircējam ir tiesības neparakstīt pieņemšanas – nodošanas aktu, ja tiek konstatēts, ka piegādātā un uzstādītā Iekārta neatbilst Līguma 1. punkta nosacījumiem, tiek konstatētas nepilnības komplektācijā, tā nefunkcionē atbilstoši savām tehniskajām īpašībām, pastāv citi Iekārtas trūkumi. Šādā gadījumā Puses sastāda aktu par Iekārtas neatbilstību un vienojas par termiņu trūkumu novēršanai. 3.11.Pircējs apņemas nosūtīt iekārtas ražotājam produkta paraugus un/vai iepakojamo materiālus testu vajadzībām, ja nepieciešams. 4. Kvalitāte un garantijas. 4.1. Pārdevējs garantē, ka Iekārtu kvalitāte un komplektācija atbilst izgatavotājfirmas noteiktajām normām un Eiropas Savienības prasībām attiecībā uz ražotāja atbildību par preces kvalitāti. Pārdevējs nodrošina piegādātajai Iekārtai kvalitātes sertifikātu, kā arī ekspluatācijas un tehnisko dokumentāciju angļu valodā. 4.2. Pārdevēja noteiktā Iekārtas garantija ir 12 (divpadsmit) mēneši no iekārtas piegādes brīža, atbilstoši Līguma pielikumā Nr.1 paredzētajām ražotāja saistībām. 4.3. Pārdevējs nodrošina veikt regulāras (atbilstoši iekārtas noslodzei) tehniskās un profilaktiskās apkopes uz savstarpēji noslēgta līguma pamata. Slēdzot šādu līgumu pārdevējs apņemas nodrošināt tehniķa ierašanos pēc izsaukuma līgumā atrunātos termiņos, kā arī informatīvo atbalstu (konsultācijas pa telefonu). 5. Strīdu atrisināšana un atbildība 5.1.Visus strīdus un nesaskaņas, kas saistītas ar šī Līguma izpildi puses risina savstarpējo pārrunu ceļā. 5.2.Ja pārrunu ceļā vienošanos nav iespējams sasniegt, tad strīdi un nesaskaņas tiek atrisināti LR tiesu iestādēs atbilstoši LR likumdošanas aktiem. 5.3. Puses apņemas neizpaust jebkuru tehnisko, finansiālo un komerciālo informāciju, kā arī citus datus, kas iegūti vai tapa zināmi Līguma darbības laikā un, kas var tikt izmantoti konkurences nolūkos. 6. Citi nosacījumi. 6.1. Puses apņemas iepriekš rakstiski informēt cita citu par iespējamām izmaiņām tās adresē, kas norādīta šajā Līgumā, nosaukumā, citos rekvizītos un juridiskajā statusā. 6.2. Pušu reorganizācija vai īpašuma formas izmaiņa nevar būt par pamatu Līguma vienpusējiem grozījumiem vai izbeigšanai, jo Līguma izpildīšanas pienākums pāriet Pušu tiesību pārņēmējiem. 6.3. Jebkuras izmaiņas un papildinājumi šajā līgumā izdarāmi tikai pēc Pušu rakstiskas vienošanās, kura no parakstīšanas brīža uzskatāma par šī Līguma neatņemamu sastāvdaļu. 6.4. Šis līgums sastādīts latviešu un angļu valodā uz 5 (piecām) lappusēm 2 (divos) identiskos eksemplāros, kuri glabājas pa vienam pie katras Puses. 6.5. Jautājumos, kas nav atrunāti šajā Līgumā, Puses vadīsies no Latvijas Republikā spēkā esošajiem tiesību aktiem. 7. Spēkā stāšanās kārtība. 7.1. Līgums stājas spēkā ar tā parakstīšanas brīdi, un darbojas līdz savstarpējo saistību izpildei. |  | ##### PURCHASE – SALE ##### AGREEMENT No. _/20__ This Agreement has been concluded in Riga, on xxx 202_ between xxxxxxxxxxx, registration No xxxxxxxxxxxxxxx, hereinafter referred to as the “Seller”, represented by its Chairperson of the Board xxxxx xxxxx, who is acting on the basis of the Articles of Association, and SIA “xxxxxxxxxxxxxxxxx”, registration No xxxxxxxxxxxxxxx, hereinafter referred to as the “Purchaser”, represented by its Chairman of the Board xxxxxxxxxxxxxxxx , who is acting on the basis of the Articles of Association, on the following: WHEREAS: 1. The Purchaser confirms that purchase of Machinery are executing on the basis of “Support for digitalisation of business processes in Latvia” No. 1.2.2.1/1/25/I/001, implemented with the framework of the specific support objective 1.2.2. “Use the advantages of digitalization for business development” of the European Union Cohesion Policy Programme 2021–2027, measure 1.2.2.1. “Support for the digitalization of processes in commercial activities”, project with number xxxxxxxxxxxxxxxxxx. 2. The Seller is approved as winner in tender procedure, …………………………………………….. 1. Subject of the Agreement. The Seller sells, delivers and installs but the Purchaser in compliance with the provisions of this Agreement purchases the equipment set of production line of production line (further in text – Machinery), whose complete set, technical characteristics, options, price, production, delivery and payment terms shall be defined in the sales offer which is submitted to the Purchaser and constitutes Annex 1 to the Agreement that is an inalienable part of the Agreement, hereinafter referred to as the “Machinery”. 1.2. Each equipment delivery Seller supply accordingly project plan fixed in Annex 2, that is an inalienable part of the Agreement 2. Prices and Total Contractual Value. The total contractual amount (a purchase price) comprises EUR XXXXXXXXXXX, 00 (XXXXXXXXXXXXXX, 00 cents). The Purchase price shall include packing, delivery and installation of the Machinery as well as single training of employees. 3. Payment and Delivery Terms. 3.1.The Purchaser undertakes to make a prepayment for the each Machinery in the amount of xx% from total amount. Production of the Machinery shall be commenced as of the moment when payment has been transferred to the Seller’s bank account. 3.2. The Purchaser undertakes to make the subsequent payment in the amount of xx% immediately following the receipt of a notification on readiness of the Machinery and receipt of an invoice thereof. Machinery will be sent from manufacturing company only after receive of mentioned payment. 3.3. The Purchaser undertakes to make the final payment in the amount of xx% within 5 (five) business days following the installation and acceptance of receipt the Machinery. When accepting the Machinery, a Deed of Conveyance shall be signed. 3.4. Date of payment shall be considered the date on which the money has been transferred in the Seller’s bank account. 3.5.The Seller undertakes to ensure production and delivery of the Machinery accordingly Annex 2 provided that obligations set in Clause 3.1 and 3.2 of this Agreement have been fulfilled and that payment set in Clause 3.2 is made without delay. 3.7.The Seller shall ensure delivery of the Machinery up to the Purchaser’s territory, installation and single training of employees. The Purchaser shall ensure the transportation of the Machinery within the Purchaser’s territory and its placement in the industrial premises (in the presence of the Seller’s representative), as well as the required power, steam, gas, drainage and compressed air connections to the Machinery. 3.8.The place of delivery and installation of the Machinery shall be xxxxxxxx. 3.9. After delivery of Machinery Seller undertakes to execute installation of accordingly Annex 2. 3.10. The ownership right to the Machinery shall be transferred from the Seller to the Purchaser at the moment of signing the DOC. The Purchaser’s authorised representative shall approve the compliance of the product with goods indicated in the waybill by signing it accordingly. 3.10.1. After signing the Deed of Conveyance, the Purchaser shall not be entitled to raise a claim with regard to the complete set. 3.10.2. Purchaser shall have the right to refuse to sign the Deed of Acceptance if the delivered and installed Machinery does not comply with Clause 1 of this Agreement, if recognized incompliance in complete set of Machinery, its not operating according to its technical features, there are other existing failures. If such circumstances exist, Parties conclude report of non-compliance and agree term for elimination of failures. - The Purchaser undertakes to send to the manufacturer of the Machinery product samples and/or packaging material for testing needs before testing of the Machinery at the factory, if necessary. 4. Quality and Warranties. 4.1. The Seller guarantees that the quality and complete set of the Machinery complies with the manufacturing company’s established standards and European Union requirements relating liability of producer for quality of goods. The Seller shall provide the delivered Machinery with a quality certificate as well as operational and technical documentation in English. 4.2. The Machinery shall be applied a warranty of 12 (twelve) months as of the moment of delivery of the Machinery in compliance with the manufacturer’s obligations provided in Annex 1 to the Agreement. 4.3. The Seller undertakes to provide regular (according to the load of the Machinery) technical and preventive maintenance services on the basis of a mutual agreement concluded. When concluding such agreement, the Seller undertakes to ensure attendance of a technician upon request within the period of time specified in the agreement as well as to provide informative support (consultations by phone). 5. Settlement of Disputes and Liability. 5.1. All disputes and disagreements that may arise in respect of execution of this Agreement the Parties shall settle by mutual negotiations. 5.2. Should the Parties fail to reach an agreement through mutual negotiations, the disputes and disagreements are subject to the legal proceedings at the judicial authorities of the Republic of Latvia according to the legislation of the Republic of Latvia. 5.3. The Parties undertake not to disclose any technical, financial and commercial information as well as any other data that have been obtained or become known during the period of validity of the Agreement and that may be used for competition purposes. 6. Other Provisions. 6.1. The Parties undertake to inform each other in writing in advance on the possible changes of either address specified in this Agreement, name, other official details and legal status. 6.2. Reorganization of the Parties or changes in their ownership may not constitute a legal basis for unilateral amendments to the Agreement or its termination as the obligation to fulfil the Agreement shall be binding to legal successors of rights of the Parties. 6.3. Any changes and amendments to this Agreement shall be implemented only upon a written agreement by the Parties, which as of the moment of signing shall be deemed an inalienable part of this Agreement. 6.4. The Agreement has been executed in Latvian and English on 5 (five) pages in 2 (two) identical copies, each copy given either Party. In case of disagreements the Latvian text of the Agreement shall prevail. 6.5. Any issues that are not provided in this Agreement, the Parties shall settle in compliance with the effectual legislation of the Republic of Latvia. 7. Implementation Procedure. 7.1. This Agreement shall come into effect as of the moment of signing it and shall be valid until complete fulfilment of the mutual obligations. |
| --- | --- | --- |
| 8. Pušu paraksti / Signatures of the Parties: |  |  |
| Pārdevējs / Seller: | Pircējs / Purchaser: |  |

Annex No. 2

For procurement No. 15042026-01

SIA Dižozols Plus

Confirmation of an independently prepared tender

Hereby, providing complete and true information,

Applicant's name, reg. No.

(hereinafter - the Applicant) in relation to the specific procurement procedure certifies that:

- The applicant has read and agrees with the content of this declaration.

2. The applicant is aware of his obligation to provide full, complete and true information in this declaration.

3. The applicant's procurement offer has been signed by the applicant's authorized person(s).

4. The applicant declares that he has submitted an offer independently of competitors (1) and without any consultations, contracts or agreements. The applicant has not had any communication with any competitor regarding:

4.1. prices;

4.2. price calculation methods, factors (circumstances) or formulas;

4.3. intention or decision to participate or not to participate in procurement (to submit or not to submit an offer); or

4.4. submission of an offer that does not meet procurement requirements;

4.5 quality, scope, specification, performance, delivery or other conditions, which can be solved independently of competitors, of the products or services to which this procurement applies.

5. The tenderer has not knowingly, directly or indirectly disclosed and will not disclose the terms of the tender to any competitor before the official date and time of the opening of tenders or the award of the contract.

- The applicant is aware that the Competition Act stipulates responsibility for prohibited agreements, providing for a fine of up to 10% of the violator's net turnover of the last financial year, and the applicant may be excluded from participation in the procurement procedure.

/_____________/  Signature*

/_____________/ Date

In the context of this declaration, the term "competitor" means any natural or legal person who is not a tenderer and who: 1) submits an offer for this procurement; 2) taking into account its qualifications, abilities or experience, as well as the offered goods or services, could submit an tender for this procurement.

* THIS DOCUMENT HAS BEEN SIGNED USING A SECURE ELECTRONIC SIGNATURE AND INCLUDES A TIME STAMP

Pielikums Nr.2

Iepirkumam Nr. 15042026-01

SIA Dižozols Plus

Apliecinājums par neatkarīgi izstrādātu piedāvājumu

Ar šo, sniedzot izsmeļošu un patiesu informāciju,

Pretendenta nosaukums, reģ. Nr.

(turpmāk – Pretendents) attiecībā uz konkrēto iepirkuma procedūru apliecina, ka:

- Pretendents ir iepazinies un piekrīt šī apliecinājuma saturam.

- Pretendents apzinās savu pienākumu šajā apliecinājumā norādīt pilnīgu, izsmeļošu un patiesu informāciju.

- Pretendenta iepirkuma piedāvājumu ir parakstījusi/šas pretendenta pilnvarotā/ās persona/s.

- Pretendents informē, ka ir iesniedzis piedāvājumu neatkarīgi no konkurentiem1 un bez konsultācijām, līgumiem vai vienošanām. Pretendentam ne ar vienu konkurentu nav bijusi saziņa attiecībā uz:

  - cenām;

  - cenas aprēķināšanas metodēm, faktoriem (apstākļiem) vai formulām;

  - nodomu vai lēmumu piedalīties vai nepiedalīties iepirkumā (iesniegt vai neiesniegt piedāvājumu); vai

  - tādu piedāvājuma iesniegšanu, kas neatbilst iepirkuma prasībām;

  - kvalitāti, apjomu, specifikāciju, izpildes, piegādes vai citiem nosacījumiem, kas risināmi neatkarīgi no konkurentiem, tiem produktiem vai pakalpojumiem, uz ko attiecas šis iepirkums.

- Pretendents nav apzināti, tieši vai netieši atklājis un neatklās piedāvājuma noteikumus nevienam konkurentam pirms oficiālā piedāvājumu atvēršanas datuma un laika vai līguma slēgšanas tiesību piešķiršanas.

- Pretendents apzinās, ka Konkurences likumā noteikta atbildība par aizliegtām vienošanām, paredzot naudas sodu līdz 10% apmēram no pārkāpēja pēdējā finanšu gada neto apgrozījuma un pretendentam var tikt piemērota izslēgšana no dalības iepirkuma procedūrā.

/_____________/  Paraksts*

/_____________/ Datums

1 Šī apliecinājuma kontekstā ar terminu „konkurents” apzīmē jebkuru fizisku vai juridisku personu, kura nav Pretendents un kura: 1) iesniedz piedāvājumu šim iepirkumam; 2) ņemot vērā tās kvalifikāciju, spējas vai pieredzi, kā arī piedāvātās preces vai pakalpojumus, varētu iesniegt piedāvājumu šim iepirkumam.

* ŠIS DOKUMENTS IR PARAKSTĪTS AR DROŠU ELEKTRONISKO PARAKSTU UN SATUR LAIKA ZĪMOGU.

Annex No. 3

For procurement No. 15042026-01

## SIA Dižozols plus

## FINANCIAL OFFER

## for participation in the procurement procedure Procurement ID No. 15042026-01

Date: ___________________ (timestamp)

The Tenderer, ____________________________, Reg. No. _______________________,
hereby offers to perform / supply the services under the procurement “Supply of integrated industrial air filtration, safety, and air humidification systems with the implementation of digitalization and artificial intelligence (AI) solutions for SIA “Dižozols Plus” needs”, in accordance with the requirements of the procurement documentation and the submitted technical specification, for the following offered price:

| No. | Item Name | Quant | Technical Specification (Summary, in accordance with the Technical Specification) manufacturer/brand/model | Price excl. VAT (EUR) | VAT (EUR) | Price incl. VAT (EUR) |
| --- | --- | --- | --- | --- | --- | --- |

Lot No. 1 Industrial dust collection, storage, safety and AI management system

| 1. | Storage Silo | 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 2. | Storage Silo Operational Accessories and Sensors | Set |  |  |  |  |
| 3. | Cyclones | 2 |  |  |  |  |
| 4. | Control of the electric equipment | 1 |  |  |  |  |
| 5. | AI-based functionalities of monitoring system | 1 |  |  |  |  |
| 6. | Spark Detection and Extinguishing System | Set |  |  |  |  |
| 7. | Heating pipe For outdoor use only | 4 |  |  |  |  |
| 8. | System installation and operational training for personnel. |  |  |  |  |  |
| Total, excluding VAT |  |  |  |  |  |  |
| VAT (21%) |  |  |  |  |  |  |
| Total, including VAT |  |  |  |  |  |  |

The applicant confirms that:

- the offered price includes all costs, including delivery, transport, installation, connection, system integration, testing, commissioning and personnel training, and other costs related to the execution of the contract;

- the offer is complete and includes all necessary components for the full operation of the system;

- the offered equipment is mutually compatible and ensures the operation of a unified system;

- additional costs not included in this offer will not be requested;

- the offer complies with all requirements of the technical specification.

| No. | Item Name | Quant | Technical Specification (Summary, in accordance with the Technical Specification) manufacturer/brand/model | Price excl. VAT (EUR) | VAT (EUR) | Price incl. VAT (EUR) |
| --- | --- | --- | --- | --- | --- | --- |

Lot No. 2 Air humidification system

| 1 | Air Humidification System (Complete Solution for All Rooms) | set |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 2 | Control and Automation System | set |  |  |  |  |
| 3 | Installation and System Integration | set |  |  |  |  |
| 4 | Testing and Training | set |  |  |  |  |
| Total, excluding VAT |  |  |  |  |  |  |
| VAT (21%) |  |  |  |  |  |  |
| Total, including VAT |  |  |  |  |  |  |

The applicant confirms that:

- the offered price includes all costs, including delivery, transport, installation, connection, system integration, testing, commissioning and personnel training, and other costs related to the execution of the contract;

- the offer is complete and includes all necessary components for the full operation of the system in all specified premises;

- the offered system ensures the required humidity parameters in all premises specified in the specification;

- additional costs not included in this offer will not be requested;

- the offer meets all requirements of the technical specification;

- if an equivalent solution is offered, it does not have lower technical parameters;

- all detailed technical documentation (brochures, drawings, CE certificates, etc.) is attached to the offer.

By submitting the tender, the Tenderer confirms that:

| No. | Requirement | Confirmation (tick) | Explanation / reference (if applicable) |
| --- | --- | --- | --- |
| 1. | The Tenderer has experience with at least 2–3 similar projects involving the supply, installation, or maintenance of industrial equipment, dust extraction systems, air filtration systems, air humidification systems, or industrial automation systems | ☐ Yes / ☐ No |  |
| 2. | The Tenderer’s average annual turnover over the last 3 calendar years is not less than the total value of the submitted offer | ☐ Yes / ☐ No |  |
| 3. | The Tenderer ensures availability of technical support and maintenance services within the territory of Latvia, providing a service response time not exceeding 24 hours on working days | ☐ Yes / ☐ No |  |

Supplier Name: ____________________________________________
Registration No.: ____________________________________________
Address: ____________________________________________
Contact Person: ____________________________________________
Phone / E-mail: ____________________________________________
Date: ___________________
Signature*: ______________________________

*THIS DOCUMENT HAS BEEN SIGNED USING A SECURE ELECTRONIC SIGNATURE AND INCLUDES A TIME STAMP