TECHNICAL SPECIFICATION No. 1

FISH PROCESSING EQUIPMENT

For procurement / request for quotation purposes. The specification incorporates the requirements stated in the supplier’s description, as well as additional technical parameters to be confirmed by the Tenderer.

Where the supplier’s description does not provide a specific value, the requirement is marked “To be specified / to be specified with technical data”. The Tenderer shall state the offered value and confirm compliance.

| No. | Technical parameter / requirement | Minimum requirement / specification | Tenderer’s offer / actual value | Compliance / remarks |
| --- | --- | --- | --- | --- |
| 1 | Equipment type | Fish smoking and thermal processing chamber; AIRMASTER UK 5000 or equivalent |  |  |
| 2 | Processing functions | Drying, hot smoking, cold smoking, hot-air cooking, preserving, defrosting |  |  |
| 3 | Intended application | Suitable for thermal processing and smoking of fish and fish products |  |  |
| 4 | Air recirculation system | High-performance recirculation system providing turbulent airflow and uniform product treatment. Volumetric flow approx. 12,000.00 m³/h, compressed-air connection R1/2”, min. 6–8 bar, min. 200 l/min |  |  |
| 5 | Air treatment unit | Integrated in the head section; high-performance fans, heating register and humidification equipment |  |  |
| 6 | Air treatment functions | Heating, cooling, drying and humidification; fresh air and smoke addition according to the process. Heating – 58.8 kW; cooling – 23 kW |  |  |
| 7 | Control system | Automatic process control with programmable temperature, relative humidity, processing time and product core temperature |  |  |
| 8 | Delta-T control | Delta-T processing or equivalent shall be available at every programme step |  |  |
| 9 | Fresh-air / exhaust-air dampers | Automatically controlled, tightly closing and pneumatically actuated |  |  |
| 10 | Safety / air evacuation at end of cycle | Process air shall be evacuated before the chamber door can be opened; fresh-air and exhaust dampers shall permit air exchange |  |  |
| 11 | Chamber construction | Stable, tightly welded modular construction |  |  |
| 12 | Chamber material | Nickel-chrome stainless steel, material No. 1.4301 (AISI 304) |  |  |
| 13 | Chamber floor | Adequate load-bearing capacity for chamber carts; integrated access ramp |  |  |
| 14 | Adjustable feet | Heavy-duty adjustable feet for compensation of uneven site floor conditions |  |  |
| 15 | Thermal insulation | Fully insulated housing using high-temperature-resistant mineral wool; 50–80 mm mineral wool insulation; thermal bridges minimised |  |  |
| 16 | Thermal bridges | Housing design shall minimise thermal bridges |  |  |
| 17 | Minimum process temperature | Minimum required process temperature capability: 160 °C |  |  |
| 18 | Heating elements | High-temperature-resistant electric heating elements integrated in the fan intake zone |  |  |
| 19 | Rated heating capacity | Rated heating capacity in kW and electrical connection requirements – 58.80 kW, 400 V/50 Hz/3 Ph |  |  |
| 20 | Humidification system | Timed water spraying via dedicated nozzles; water mist is vaporised by heat from the heating register |  |  |
| 21 | Water connection | Hot-water connection recommended; R 1”, minimum water flow pressure 4 bar |  |  |
| 22 | Smoke supply | Provision for controlled smoke addition according to the selected programme; smoking material: wood chips (particle size 4–12 mm) |  |  |
| 23 | Fans | High-performance fans, at least 2 pcs.; 11.0 kW |  |  |
| 24 | Airflow distribution | Turbulent and uniform airflow throughout the treatment chamber; volumetric flow approx. 12,000.00 m³/h |  |  |
| 25 | Loading system | Two mobile floor-mounted chamber carts |  |  |
| 26 | Cart dimensions (corresponding to equipment max. dimensions) | 1000 × 1000 × 2000 mm |  |  |
| 27 | Alternating airflow damper system | Enables variable supply of airflow into the treatment chamber |  |  |
| 28 | Cooling evaporator | Two-circuit cooling system consisting of a circulating-air cooler with max. capacity 7 kW connected to the fresh-air intake, and external cooling evaporators located outside, with max. capacity 16 kW |  |  |
| 29 | Lighting | Includes chamber lighting with a switch |  |  |
| 30 | Door stopper | Mounted on the entry side to limit the opening angle |  |  |
| 31 | Door construction | Front fully insulated block door with heavy-duty stainless-steel hinges |  |  |
| 32 | Door interlock system | Safe automatic door opening/closing and interlocking system ensuring safe operation of the equipment and preventing unauthorised opening of the door during the technological process. |  |  |
| 33 | Door seal | Heat-resistant silicone rubber gasket providing a secure seal |  |  |
| 34 | Emergency opening from inside | Door shall be capable of being opened from inside in accordance with applicable safety requirements |  |  |
| 35 | Door handing | DIN right-hand configuration |  |  |
| 36 | Status indication | Eye-level signal lamp indicating machine operating status |  |  |
| 37 | PLC / automation | Industrial PLC/control system; manufacturer, model and control panel type shall be stated; screen size min. 7”, LCD touch panel |  |  |
| 38 | Recipe / programme management | Ability to create, store, modify and recall process programmes. Memory capacity for 250 programmes of 20 steps each |  |  |
| 39 | Temperature control accuracy | Tenderer shall state the control range and guaranteed control accuracy |  |  |
| 40 | Relative humidity control | Tenderer shall state the RH control range and accuracy |  |  |
| 41 | Core temperature control | Product core-temperature control shall be provided; 1 core-temperature sensor |  |  |
| 42 | Total connected electrical load | Tenderer shall state the total connected load in kW |  |  |
| 43 | Power | Max. 73 kW |  |  |
| 44 | Water consumption | Max. 100 l/min for cleaning |  |  |
| 45 | Compressed-air requirements | R 1/2”, 8 bar, 300 l/min during cleaning |  |  |
| 46 | Drain connection | Tenderer shall state drain connection diameter, drainage requirements and wastewater volume |  |  |
| 47 | Exhaust / flue connection | 1 × DN 200, exhaust air volume max. 1200 m³/h, one separate exhaust-air chimney DN 250 for each machine |  |  |
| 48 | Overall dimensions | Maximum permissible equipment dimensions, taking into account the space available for installation: L × W × H – not more than 2500 × 2200 × 3800 mm. |  |  |
| 49 | Net equipment weight | Tenderer shall state equipment weight in kg |  |  |
| 50 | Required installation area (optional) | L × W × H = 2500 × 2200 × 3800 mm |  |  |
| 51 | Ambient operating conditions | Minimum temperature 10 degrees, maximum humidity 90% |  |  |
| 52 | Cleaning and hygienic design | Food-industry hygienic design enabling regular washing and cleaning of the equipment; integrated automatic washing/self-cleaning system. |  |  |
| 53 | Food-contact / hygienic compliance | Materials and components shall comply with applicable EU food-hygiene requirements |  |  |
| 54 | Safety functions | Emergency stop, door safety interlock, overtemperature protection and other manufacturer-specified safety functions |  |  |
| 55 | CE conformity | Equipment shall bear CE marking and be supplied with the applicable EU Declaration of Conformity and technical documentation |  |  |
| 56 | Available technical documentation | Operating, maintenance and safety manuals; electrical/pneumatic schematics; spare-parts list |  |  |
| 57 | Warranty | Warranty period – not less than 24 months from signing of the acceptance and handover certificate and commissioning of the equipment. The warranty applies to the equipment and its components, except for consumable/wear parts clearly identified in the manufacturer’s documentation as requiring regular replacement during operation. The Tenderer shall specify warranty exclusions in the tender. |  |  |
| 58 | Service and spare parts | The Tenderer shall ensure availability of warranty service in Latvia or specify the arrangements for providing service in Latvia. The Tenderer shall state the procedure for submitting warranty claims, initial response time, service location and spare-parts supply arrangements. |  |  |
| 59 | Delivery, installation and commissioning | Delivery, installation, connection, testing, commissioning and operator training are included in the quoted price |  |  |
| 60 | Component origin / quality | Equipment components shall be suitable for industrial operation, mutually compatible and compliant with applicable EU safety and conformity requirements. |  |  |
| 61 | Energy efficiency | Maximum power consumption is 73 kW. Average consumption is 30%. Cold-smoking cycle lasts 6 hours – 130 kW |  |  |

Procurement note:

- The Tenderer must complete the column “Tenderer’s offer / actual value”, stating the specific technical parameter of the offered equipment or confirming fulfilment of the requirement. In the column “Compliance / remarks”, the Tenderer shall state “Complies” or provide the necessary explanation. If a requirement is objectively not applicable to the proposed technical solution, “N/A” shall be stated together with justification.

- The Tenderer shall attach to the tender the manufacturer’s technical documentation, data sheet, catalogue or another document issued by the manufacturer, to an extent sufficient for the Contracting Authority to verify the compliance of the offered equipment with the requirements set out in the technical specification.